TENDER SPECIFICATIONS

CRCI is composed of:

Unit (in metallic box), including:

2 Industrial Controllers modules. Each module is composed of:

Industrial Controller:

ABB Controller CM30.

Display: color 1/4 VGA TFT, liquid crystal display (LCD) with built-in backlight.

Language: English, German, French, Italian and Spanish.

Operator keypad: 6 tactile membrane keys.

Password protection: Basic/Advanced user-assigned password protection.

Trend display:

Recording of 2 variables.

Configurable sample rate (1 second to 5 minutes).

Control parameters:

Proportional band: 0 to 999.9 %.

Integral: 0 to 10000 s.

Derivative: 0 to 999.9 s.

Manual Reset 0.0 to 100 %.

Autotune: On-demand calculation of control settings.

Process alarms:

Number: 8.

Types: High/Low process and High / Low latch.

Acknowledgement: Via front panel keys or digital signals.

Source: Fully configurable (for example, PV, analog input, math block inbuilt, OP control loop deviation).

Hysteresis: Level and time.

Alarm enable: Enable/Disable of individual alarms via a digital signal.

Analog inputs:

Number: 2.

Complete configurable process input: voltage (V or mV), current (mA), resistance (ohms), Thermocouple, 3-Wire RTD, frequency, pulse, volt-free digital input and 24V digital input.

Selectable set points via software (remote from the computer) or front panel.

Analog/Digital output:

Number: 1.

Type: configurable as analog or digital pulse.

Analog range: 0 to 20 mA (programmable).

Control output types (Configurable through software):

Current proportioning/Voltage proportioning (using resistor).

Time proportioning.

On/Off.

Motorized valve with feedback.

Motorized valve without feedback.

Split output with combinations of relay digital O/P and current O/Ps.

Relays:

Number: 2.

Type: N/O.

Contact rating: 5A, 240V.

Communication (PC): RS-485 connector (Profibus DP).

Set Point Generator module:

2 Voltages selectable through a switch, which allows generate a step signal.

Amplitude: 0 to 10V DC.

All process variables are accessible as analog signals at lab jacks.

Possibility of connection of external instruments via lab jacks (for example: line recorder, oscilloscope, etc).

Interface card, with RS-485 connector, to connect the industrial controllers through a field bus (Profibus DP) with the computer (PC).

Software: it includes the OPC server drivers and the Control Configuration Software for the Industrial Controllers.

Cables and Accessories, for normal operation.

This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

- Dimensions: 490 x 330 x 310 mm approx. (19.29 x 12.99 x 12.20 inches approx.). - Weight: 12 Kg approx. (26.4 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Installation, connection and familiarization with the software.

2.- Familiarization with the navigation of an industrial controller (operator page screen, trend display screen, etc).

3.- Setting up an industrial controller to accommodate the analog inputs/outputs signals types and engineering and electrical units.

4.- Function of a digital industrial controller.

5.- Configuration of analog inputs/outputs signals of the industrial controller (electrical range, engineering units, fault detector, etc).

6.- Setting alarms and diagnostics generation.

To learn and to familiarize with the operation and structure of a process control system under Profibus DP:

7.- Layout of a field bus system using the Profibus DP field bus.

8.- Study of OPC (OLE for Process Control) server function.

9.- Configuration of an industrial controllers networking through the software: Master and slave assignment.

10.- Reading control variables and PID parameters and displaying them on computer (PC) monitor.

11.- Scaling displays.

12.- Setting of an industrial controller using the front panel (configuration level, parameter level, operation control level, etc.).

13.- Remote setting of an industrial controller using the control configuration software (configuration level, parameter level, operation control level, etc.).

Optional

CRCI/ICAI. Interactive Computer Aided Instruction Software System:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.