Tender Specifications

Unit mounted on an anodized aluminum frame and panels in painted steel.

The unit includes wheels to facilitate its mobility.

Diagram in the front panel with distribution of the elements similar to the real one.

Centrifugal fan, with speed control:

Max. volumetric flow rate: 1800 m³/h.

Max. motor power: 750 W.

It includes a plate with one orifice at the discharge side of the fan to carry out air jet dispersion practical exercises, orifice diameter: 30 mm.

Long smooth test pipe, inner diameter 81.4 mm; length: 2500 mm approx. It includes:

Deflector in the inlet to prevent air-separation from the wall of the pipe and reduce the formation of vortices.

Ten pressure tappings to measure the pressure gradient along the pipe.

Five stations to use the transverse Pitot tube and to determine the boundary layer growth and the development of velocity profile.

Two Pitot tubes:

Pitot tube located at the discharge of the fan, attached and properly protected. It can be situated at different distances from the discharge orifice to study free jet velocity profiles:

Horizontal: -150…150 mm.

Vertical: -150…150 mm.

Pitot tube can be located in any of the different points of the test tube to perform an air flow analysis of the velocity profile inside the pipe section. This Pitot tube can also be located at different distances in the pipe to demonstrate the boundary layer growth:

Vertical: -24…38 mm.

Iris diaphragm, diameter: 40 - 97 mm. It includes two pressure tappings.

Orifice plate, diameter: 50 mm.

Two nozzles with different diameters, diameters: 50 and 70 mm.

Two bends and an angle may be fitted to the test pipe to compare pressure losses in fittings.

Multi-tube inclinable manometer, which can be inclined to increase its resolution. It includes: 20 water tubes with a length of 500 mm and inner diameter of 6 mm, and a tank made of PMMA to fill it with water.

Control panel:

Main switch.

ON/OFF Fan switch.

Fan speed controller.

Fan speed display.

Cables and Accessories, for normal operation.

Manuals: This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

Optional Models and Accessories: (Not included)

- HECA-1. Venturi Tube.

- HECA-2. Accesory to measure boundary layers on a flat plate.

- HECA-3. Accesory to study the Drag Forces.

Dimensions:  3000 x 600 x 1400 mm approx. (118.10 x 23.62 x 55.11 inches approx.)  Weight: 90 Kg approx. (198 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.-  Utilization of manometers to measure pressure drops.

2.-  Measurement of air flow rate with a Pitot tube.

3.-  Measurement of air flow rate with an iris diaphragm.

4.-  Measurement of air flow rate with an orifice plate.

5.-  Measurement of air flow rate with a nozzle.

6.-  Determination of pressure losses in straight pipes and pipe friction factor.

7.-  Determination of pressure losses in bends and angles and the resistance coefficient for each bend and angle.

8.-  Determination of the velocity profile in a pipe through which air is flowing.

9.-  Determination of the velocity profile in a free jet using the Pitot tube.

10.-Understanding the use of Reynolds' numbers.

11.-Demonstration of the boundary layer growth.

Additional practical possibilities (It requires the optional accessory HECA-1):

12.-Measuring the air flow via a Venturi nozzle.

13.-Conservation of energy and law of continuity.

Additional practical possibilities (It requires the optional accessory HECA-2):

14.-Study of the boundary layer on a flat plate.

15.-Study of the velocity profile.

Additional practical possibilities (It requires the optional accessory HECA-3):

16.-Study of the drag forces in different shapes.

Optional

HECA/ICAI. Interactive Computer Aided Instruction Software System:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc...so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.