# TENDER SPECIFICATIONS

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Centrifugal pump: 0.37 KW, 30-80  l/min at 20.1-12.8m., single-phase 220V/50Hz. or 110V/60Hz.

Stainless steel impeller.

Tank capacity: 140  l. approx.

Flowmeter.

Membrane type flow adjusting valve.

Safety switch ON/OFF.

Supports for accommodating the test module.

This unit incorporates wheels for its mobility.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Dimensions: 1000 x 600 x 700 mm. approx.  (36.36 x 23.62 x 27.55 inches approx.)  Weight: 40 Kg. approx. (88.18 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

- Flow measurement.