# TENDER SPECIFICATIONS

Iron frame with ventilation grill.

Stainless steel water tank (volume: 10.5  l.). It has one water inlet, one water outlet and two vapour outlets.

Vapour generation: 10-12 Kg.v.h.

ON switch.

Indicator for the heating element switching on/off.

Pressure gauge to determine the pressure inside the generator. Range: 0-10 Bar.

Pressure working range: 0- 8 Bar.

Electric heating element: 6500 W.

Automatic admission valve.

Safety pressure switch (8 Bar).

Safety valve: 10 Bar.

Two safety level switches (one for minimum level and the other for maximum level).

Cables and Accessories, for normal operation.

Manuals:	This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

Dimensions: 700 x 500 x 800 mm. approx. (27.55 x 19.68 x 31.49 inches approx.)     Weight: 60 Kg. approx. (132.27 pounds approx.).