# TENDER SPECIFICATIONS

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Anodized aluminum structure and panels of painted steel.

3 Hydrometers of resolution 0.002ºSG:

Hydrometer 0.8ºSG - 1ºSG

Hydrometer 1ºSG – 1.2ºSG

Hydrometer 1.2ºSG – 1.4ºSG

Two hydrometer jars of 450 x 50 mm.

Aneroid barometer, range: 973 – 1047 mbar.

Thermometer with a range between -10 and 50ºC.

Pycnometer of 50 ml.

Parallel plates capillary module.

Capillary tubes module with tubes of different size: 5 mm, 4 mm, 3 mm, 2.2 mm, 1.7 mm and 1.2 mm.

Two falling sphere viscometer tubes of 300 x 40 mm, with marks at 0, 25, 175, 200 and 220.

Set of stainless steel balls of different sizes: 3.175 mm, 2.381 mm and 1.588 mm.

Variable scale lever balance to be used with the Archimedes module, up to 310 gr.

Archimedes module: displacement vessel, bucket and cylinder.

Graduated cylinder made of glass (250 ml.)

2 Beakers made of glass (600 ml.)

Digital chronometer.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Dimensions: 850 x 500 x 800 mm. approx. (33.46 x 19.68 x 31.49 inches approx.).  Weight: 20 Kg. approx. (44 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.- To study the effect of capillary elevation between flat plates.

2.- To study and measure the effect of capillary elevation inside capillary tubes.

3.- To study and verify the Archimedes principle using a bucket and cylinder with a lever balance.

4.- To measure the fluid density and relative density of a liquid using a hydrometer and using a density bottle.

5.- To measure the atmospheric pressure using a barometer.

6.- To measure the fluid viscosity using a falling sphere viscometer.

7.- To measure the fluid temperature using an alcohol thermometer.

8.- Measuring of liquid levels.

Optional

FME35/ICAI. Interactive Computer Aided Instruction Software System:

This complete software package consists of an Instructor Software (INS/SOF) totally integrated with the Student Software (FME35/SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.

BDAS. Basic Data Acquisition System and Sensors:

- BDAS/BFA:

Base Module:

Anodized aluminum frame and panel made of painted steel.

Main elements made of stainless steel.

This unit has wheels to facilitate its mobility.

Sensors (only the sensors that correspond according to the purchased module/s will be included):

2 Pressure sensors, range: 0-100 PSI (0-7 bars).

2 Differential pressure sensors: they measure displacement through two manometers with a range of up to 1 meter, scale: 0.1 mm.

Flow sensor for high working flows, range: 5-150 l/min.

Flow sensor for low working flows, range: 1.5-30 l/min.

Piezoresistive force sensor. It measures from 0 Kg to 1.5 Kg and converts the value into Newton.

Force sensor to calculate the braking torque of turbines. It measures from 0 Kg to 2 Kg and converts the value into Newton.

Optical speed sensor to measure the speed of turbines.

Measurements of the speed and torque of the pumps.

Dimensions: 300 x 550 x 1200 mm. approx. (11.81 x 21.65 x 47.24 inches approx.).  Weight: 10 Kg. approx. (22 pounds approx.).

Data Acquisition System:

Data Acquisition Electronic Box, with connectors for the different sensors.

PCI Express data acquisition board (National Instruments) that is lodged in a slot of the computer. PCI Express bus.

Data acquisition software.

This system enables:

To represent the system responses curves in real time.

To record all the measurement values and results in a file.

To plot the characteristic curves.

To calibrate the sensors that take part in the process.

Dimensions: 490 x 330 x 310 mm. approx. (19.29 x 12.99 x 12.20 inches approx.) Weight: 10 Kg. approx. (22 pounds approx.).

FME35/BDAS-BFI: Specific Software and accessories for FME35.

These accessories are used to adapt each module in order to operate them and connect them to the Base Module (BDAS/BFA) easily.