Tender Specifications

The “AV-ECD” unit is mounted in a metallic box, which can be assembled in vertical position or used on the surface of a table.

The "AV-ECD" can work alone, but the electronic control device also allows to be connected to some sensors of the "AV-S" unit and some actuators of the "AV-A" unit in order to work with real sensors and actuators.

All circuits include several test points, with a standard 2 mm lab socket, to access and analyze different circuit points.

The sensor and actuator circuits allow the simulation of failures in the components, inserted with toggle switches. The system failures are indicated by the malfunction indicator lamp (MIL). The sensors and actuators can work individually or with other related sensors and actuators.

The circuits included in the “AV-ECD” unit are divided into:

Electronic control unit (ECU):

Communication with sensors and actuators through CAN or LIN network protocols or through digital or analog signals.

ECU self-diagnostic to detect failures.

Flashing of the ECU with two engine configurations: naturally aspirated and turbocharged.

Emulated sensor circuits:

Manifold absolute pressure sensor.

Knock sensor.

Accelerator pedal position sensor.

Intake air temperature sensor.

Refrigerant temperature sensor.

Exhaust Gas Recirculation (EGR) temperature sensor.

Oxygen or Lambda sensor.

Mass air flow sensor.

Crankshaft position sensor.

Camshaft position sensor.

Fuel tank pressure sensor.

Emulated actuator circuits:

Canister-purge solenoid valve.

Turbocharger boost control solenoid.

Exhaust Gas Recirculation (EGR) valve.

Spark plug.

Fuel injector.

Electronic throttle valve.

Engine cooling fan.

Cables and Accessories, for normal operation.

Manuals: This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

Dimensions: 490 x 330 x 310 mm approx. (118.10 x 70.86 x 66.92 inches approx.)  Weight: 12 Kg approx. (1410 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

Electronic control unit (ECU):

1.-  Communication with sensors and actuators through CAN or LIN network protocols or through digital or analog signals.

2.-  Performance of an ECU self-diagnostic to detect failures.

3.-  Understanding the Manifold Absolute Pressure (MAP) sensor signal.

4.-  Knock sensor signal.

5.-  Function of the fuel tank pressure sensor and the canister-purge solenoid valve.

6.-  Refrigerant temperature sensor signal.

7.-  Function of the Exhaust Gas Recirculation (EGR) temperature sensor.

8.-  Function of the Accelerator Pedal Position (APP) sensor.

9.-  Oxygen or Lambda sensor signal.

10.-Mass air flow sensor signal.

11.-Intake air temperature sensor signal.

12.-Crankshaft position sensor signal.

13.-Camshaft position sensor signal.

14.-Checking failures in the sensor signals.

15.-Turbocharger boost solenoid control signal.

16.-Function of the Exhaust Gas Recirculation (EGR) valve.

17.-Understanding the electronic throttle valve function.

18.-Function of the fuel pump.

19.-Spark plug control signal.

20.-Fuel injector control signal.

21.-Engine cooling fan control signal.

22.-Checking failures in the actuator signals.

23.-ECU self-diagnostic function.

24.-Flashing the ECU with naturally aspirated engine configuration.

25.-Flashing the ECU with turbocharged engine configuration.

- Several other exercises can be done and designed by the user.

Optional

AV-ECD/ICAI. Interactive Computer Aided Instruction Software System:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.