TENDER SPECIFICATIONS

The Microwave Trainer “EMI” is composed of:

Electronic console (in metallic box):

Gunn oscillator connector. Power meter connector. Power meter display. SMA connector to slotted line. SWR measurement display.

Power meter:

Based on Thermistor. Wide frequency range: 10 to 12000 MHZ.

Typically level range: -55 dBm to +18 dBm. Stability over temperature. Slope:-25mV/dB.

Connector to electronic console.

Slotted line:

Tunnel Diode detector mounted. Frequency range: 2 to 18 Ghz. VSWR (max.): 3.5:1.

Maximum input power: 100mW (20dBm). Work in the quadratic detection zone.

Designed for 8.2 to 12.4 GHz (X-band). Longitudinal movable Diode detector holder.

Millimeter ruler. Waveguide in the standard size WR-90.

SMA connector to electronic console.

Gunn oscillator (Microwave signal generator):

Gunn oscillator diode. Frequency band: X band (fixed at 10.525Ghz).

Power output: 10dBm. Waveguide in the standard size WR-90.

Connector to electronic console.

Broad-wall Waveguide Directional Coupler:

Designed for 8.2 to 12.4 GHz (X-band). 3 ports (input, output and coupled).

Waveguide in the standard size WR-90.

Cross-guide Waveguide Directional Coupler:

Designed for 8.2 to 12.4 GHz (X-band). 4 ports (input, output, isolated and coupled). Waveguide in the standard size WR-90.

Hybrid Tee:

Designed for 8.2 to 12.4 GHz (X-band).

3dB coupler. 4 ports (2 co-linear, sum and difference). Waveguide in the standard size WR-90.

6dB Fixed attenuator:

Designed for 8.2 to 12.4 GHz (X-band). Fixed attenuator at 6 dB. Waveguide in the standard size WR-90.

15dB Fixed attenuator:

Designed for 8.2 to 12.4 GHz (X-band). Fixed attenuator at 15 dB. Waveguide in the standard size WR-90.

Vertical variable attenuator:

Designed for 8.2 to 12.4 GHz (X-band). Precision micrometer. Waveguide in the standard size WR-90.

Horizontal variable attenuator:

Designed for 8.2 to 12.4 GHz (X-band). Precision micrometer. Waveguide in the standard size WR-90.

Termination load (Dummy load):

Designed for 8.2 to 12.4 GHz (X-band). Adapted load. Waveguide in the standard size WR-90.

2 Short circuit terminations.

2 Horn antennas:

Designed for 8.2 to 12.4 GHz (X-band). Sectorial horn flared in the direction of the electric plane (H-plane).

Waveguide in the standard size WR-90.

Adjustable termination:

Designed for 8.2 to 12.4 GHz (X-band). Movable short circuit. Precision micrometer. Waveguide in the standard size WR-90.

Microwave Absorber Plate: Plate with resistive material.

Microwave Reflector Plate: Plate with metallic material.

4 Tower antennas with adjustable height, including each one: waveguide holder and degree disc with position indicator.

10 Quick release fasteners for an easy operation with the waveguide devices.

Cables and accessories, for normal operation.

Manuals: This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

Dimensions: 800 x 600 x 450 mm. approx. (31.49 x 23.62 x 17.72 inches approx.).     Weight: 20 Kg. approx. (44 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.-Familiarization with the microwave trainer.

2.-Power emission measurement.

3.-Study of different fixed attenuators and variable attenuators.

4.-Calibration of variable attenuators.

5.-Wavelength, frequency and SWR measurement with the waveguide slotted line.

6.-Basic principles of Smith chart.

7.-Calculation of impedance, admittance and reflection coefficient, with the Smith chart.

8.-Calculation of impedance, admittance and reflection coefficient for different terminations, with the Smith chart.

9.-Comparison between matched and mismatched loads.

10.-Study of the Broad-wall Waveguide Directional Coupler.

11.-Study of the Cross-guide Waveguide Directional Coupler.

12.-Study of the Hybrid Tee.

13.-Measure of power emission in free space.

14.-Measure of wavelength, frequency and SWR measurement in free space.

15.-Radiation pattern of a horn antenna.

16.-Study of gain and directivity of a horn antenna (dBi).

17.-Reflectance and absorbance of metallic and resistive plates.