TENDER SPECIFICATIONS

Items supplied as standard

1 CODITEL. Unit:

System is mounted on a desktop box.

Diagram in the front panel of the unit with the same structure as the real hardware.

Provides access to all main signals.

Follows all the recommendations of CCITT (Cosultive Comitte International Telephony and Telegraphy). Structure is based on the standard for digital commutation systems. Time division multiplexing(TDM) and Pulse code Modulation (PCM) principles.

Communication based on a temporal commutator that supports up to fifteen complete internal communications with two MIC ways, one inlet way and one outlet way. Standard  MIC  system  of  32  channels  with  dynamic  assignament  and  frame synchronization. 30 channels used for voice transmission and 2 used for signalling and synchronism. Signal conversion using standard codec, 8 KHz of sample rate, 8 bits per sample and 64Kb/s per channel and 2,048 Mb/s system frame.

Internal  hardware  consists  of  two  electronics  boards:  an  analog  board  with  all integrated and discrete  analog  elements  and  a  digital  board  based  on  FPGA technology.

Structure integrated with all the internal common elements in a telephone system: subscriber´s line, tone generator, base of times, filtering stage, switching matrix, number decoders and control memory.

Two subscriber`s lines implemented with discrete elements (hybrid coils, ring circuit, supply circuit).

Two  subscriber`s  lines  implemented  with  integrated  elements  Subscriber`s  Line Interface Circuits (SLIC).

Functions as battery feed, overvoltage protection, ringing, coding and decoding, supervision, signalling, 2-4 wires conversion and test (BORSCHT functions) covered, in each line.

High precision digital tone synthetizer 400 Hz (dial invitation tone, call recapturing tone and busy tone) using Direct Digital Synthetizer (DDS).

Four commercial receptors (telephones),  two used for signaling by pulses and other two for tones or Dual Tone Multi Frequency Signalling (DTMF).

Switching memory matrix, control memory and base of times implemented with FPGA technology with high stability and fiability.

Standard RS232 protocol for communication between the PC and the trainer.

2 CODITEL/CCSOF. Computer Control and Graphic Visualization Software:

Compatible with actual Windows operating systems. It is executed under windows operating system 2000 or higher.

Compatible with the industry standards.

In combination with the hardware, Coditel Software is supplied with the system.

Coditel software has a user friendly graphical interface that provides totally control and visualization of the telephone system.

- Cables and Accessories, for normal operation.

4  Manuals:

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control  Software,  Starting-up,  Safety,  Maintenance,  Calibration  & Practices Manuals.

Dimensions: 490x330x310 mm. approx. Weight:20 Kg. approx.

* References 1 to 4: CODITEL + CODITEL/CCSOF + Cables and Accessories + Manuals are included in the minimum supply,

enabling a normal operation.

EXERCISES AND PRACTICAL POSSIBILITIES

Some Practical Possibilities of the Unit:

- To study the main actions and signals involved in a digital commutation.

- To study the dynamic channel assignment and temporal switching.

- To study the standards for audio conversion.

- To establish of a communication between some channels step by step.

- Visual monitoring of the main states that a line goes through during a call.

- To configure lines as only receiver, transmitter, receiver / transmitter.

- To test of the conversion from 2 to 4 wires.

- To study the electric stages when  the user makes actions over the telephone.

- To study the signals involved when dialing by pulses.

- To study the signals involved when dialing by tones.

- To study the tone signal.