TENDER SPECIFICATIONS

The application includes the following modules:

• N-ALI02. Domestic Main Power Supply.

Voltage supply (Single-Phase): 230 VAC, PH+N+G.

ON-OFF removable key.

Output Voltage Connections:

Two Single-Phase: 230 VAC.

Single-Phase supply wire connecting plug.

Differential magnetothermal, 2 poles, 25 A, 30 mA AC 6 KA.

• N-ALI03. AC Auxiliary Power Supply.

Voltage supply (Single-Phase): 230 VAC PH+N+G.

Output voltage:

Single-Phase 24 VAC/12 VAC.

24 VDC.

0 - 24 VDC through potentiometer.

• N-DET13. Wireless Intrusion Detector RF.

Vision angle: 90º.

Frequency: 433.92 MHz.

Voltage supply: battery of 3.6 VDC.

Max. Area 12 x 12 m.

Status LED.

• N-DET14. Wireless Panic Push-button RF.

Frequency 433.92 MHz.

Voltage supply: battery of 12 VDC.

Battery status LED.

Working temperature 0 – 49ºC.

Dimensions: 32 x 53 x 17 mm.

• N-DET15. Wireless 1-channel Receptor RF.

Communication with 10 transmitter modules.

Frequency: 433.92 MHz.

Voltage supply: 12 VDC.

One alarm output.

One tamper output.

Dimensions: 110 x 63 x 25 mm.

• N-SEL01. Light Signaling Beacons.

Two pilots, yellow and red.

Voltage supply: 230 VAC.

• N-TIM05. Bell + Buzzer.

Voltage supply: 230 VAC.

Bell sound 70 dB.

Buzzer sound 80 dB.

• All necessary cables to realize the practical exercises are included.

Cables and Accessories, for normal operation.

Manuals:

This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

- Dimensions: 640 x 320 x 670 mm approx. (25.19 x 12.59 x 26.37 inches approx.) - Weight: 60 Kg approx. (132 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.-   Checking and measuring on the main power supply.

2.-   Checking the working of  the auxiliary power supply.

3.-  Checking the working of the bell + buzzer module using as acoustic alarm.

4.-  Checking the working of the light signaling beacons module, using as visual alarm.

5.-  Checking the working of  the wireless emergency pushbutton.

6.-  Use of the alarms with the wireless intrusion detector.

7.-  Real application of a wireless intrusion detection system.

Optional

AEL-AD23/ICAI. Interactive Computer Aided Instruction Software System:

This complete software package consists of an Instructor Software (INS/SOF) totally integrated with the Student Software (AEL-AD23/SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.