TENDER SPECIFICATIONS

The trainer includes the following modules:

• N-VPS01. AC 3PH Variable Power Supply.

ON-OFF removable key.

Input Voltage: 400 VAC, 3PH+N.

Output Voltage:

0…450 VAC.

0…24 VDC.

Maximum current: 2A.

Frequency: 50/60 Hz.

Wheel to regulate the voltage.

Three lamps.

Measurement commutators:

One commutator to measure the voltage:

6 positions: L1-N ; L2-N ; L3-N ; L1-L2 ; L1-L3 ; L2-L3.

One commutator to measure the current:

3 positions: L1, L2, L3.

Three-Phase supply hose with IP44 3PN+E 32A 400V connecting plug.

Differential magnetothermal, 4 poles, 25A, 300mA AC 6KA.

• N-MED09. AC Ammeter (0-2.5 A). (2 units).

Measurement range: 0-2.5 A.

Terminals:

Measurement Terminals.

Ground Terminal.

• N-MED22. AC Voltmeter (0-400 VAC).

Measurement range: 0-400 VAC.

Terminals:

Measurement Terminals.

Ground Terminal.

• N-MED32. 1-Phase Wattmeter 230 V.

Nominal voltage 230 VAC.

Terminals:

Measurement Terminals.

Ground Terminal.

• N-MED65. Digital Multimeter.

This module has a digital multimeter of 3 ½ digits, with double-jack ending cables of 4 mm to facilitate interconnections.

With this digital multimeter we will be able to measure:

Voltage.

Current.

Resistance.

Capacitors capacity.

Temperature.

• TRA31. 1kVA Single-Phase Transformer.

Nominal supply voltage: 230 VAC.

Nominal output voltage: 230 VAC

Nominal power: 1kVA.

• N-REV. Single phase variable resistor.

Variable resistor of 150 Ohm.

Maximum power: 500 W.

Potentiometer.

Terminals:

Three terminals to choose all resistance or variable resistance.

Fuse: 2 A.

• N-IND. Inductance module.

Value: 1.4H.

Current fuse: 2A

Selector:

Position 0: circuit opened.

Position 1: circuit closed.

• N-CON. Capacitor Module.

Capacitor value: 7 uF.

Nominal Voltage: 230 VAC.

Selector:

Position 0: circuit opened.

Position 1: circuit closed.

• All necessary cables to realize the practical exercises are included.

Cables and accessories, for normal operation.

Manuals:

This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

Dimensions: 1380 x 840 x 2010 mm. approx. (54.33 x 33.07 x 79.13 inches approx.).   Weight: 50 Kg. approx. (110 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Calculation of transformer ratio.

2.- Open circuit test.

3.- Short circuit test.

4.- Determination of equivalent circuit.

5.- Drop voltage with resistive load.

6.- Drop voltage with inductive load.

7.- Drop voltage with capacitive load.

8.- Study the efficiency of the transformer.

9.- Determination of load which allows the maximum efficiency of the transformer.

AEL-SPTT/ICAI. Interactive Computer Aided Instruction Software System:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager –ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft –ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

-ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc...so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

User Data Base Management.

Administration and assignment of Workgroup, Task and Training sessions.

Creation and Integration of Practical Exercises and Multimedia Resources.

Custom Design of Evaluation Methods.

Creation and assignment of Formulas & Equations.

Equation System Solver Engine.

Updatable Contents.

Report generation, User Progression Monitoring and Statistics.

-ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired technical knowledge.

Innovative features:

Student Log-In & Self-Registration.

Existing Tasks checking & Monitoring.

Default contents & scheduled tasks available to be used from the first session.

Practical Exercises accomplishment by following the Manual provided by EDIBON.

Evaluation Methods to prove your knowledge and progression.

Test self-correction.

Calculations computing and plotting.

Equation System Solver Engine.

User Monitoring Learning & Printable Reports.

Multimedia-Supported auxiliary resources.