Tender Specifications (for main items)

1 AEL-SST-02. Unit.

Required basic equipment of AEL-SST-01:

• N-VPS01. AC 3PH Variable Power Supply.

ON-OFF removable key.

Input Voltage: 400 VAC, 3PH+N.

Output Voltage:

0…450 VAC.

0…240 VDC.

Maximum current: 2 A.

Frequency: 50/60 Hz.

Wheel to regulate the voltage.

Three lamps.

Measurement commutators:

One commutator to measure the voltage:

6 positions: L1-N ; L2-N ; L3-N ; L1-L2 ; L1-L3 ; L2-L3.

One commutator to measure the current:

3 positions: L1, L2, L3.

Three-Phase supply hose with IP44 3PN+E 32A 400V connecting plug.

Differential magnetothermal, 4 poles, 25A, 300mA AC 6KA.

• N-EALD. Network Analyzer Unit with Computer Data Acquisition.

ON-OFF switch.

Supply voltage: 400 VAC.

Input terminals: Input connection with the measurement point.

Output terminals: Output connection with the measurement point.

Digital outputs: Three digital outputs are used for pulses or alarms, or for combining both.

RS-485 Communication port.

Fuses: 3x10 A.

Network Analyzer Display. It shows:

Active, reactive and apparent power.

Active, reactive and apparent energies.

Lines and phase currents.

Line and phase voltages.

Frequencies.

Power Factor.

• N-BUS08. Distribution Busbar Module (4 units).

Supply Voltage: 230 VAC.

Switch ON/OFF.

Local/remote control switch.

Generation double busbar module.

Four Sliding commutators of disconnectors and circuit breaker.

State indication lamps:

Two yellow lamps to indicate the state of busbars.

Three red lamps to indicate the state of the switch and the disconnectors.

Communication connector SUB-D of 62 pins.

• N-BUS09. Coupling busbar Module.

Coupling double busbar module.

Local/remote control switch.

Sliding commutators of disconnectors and circuit breaker.

State indication lamp.

ON-OFF switch.

Communication connector SUB-D of 62 pins.

• N-REVT/1K. 1kW Three-Phase Resistors Module.

Parallel, series, star and delta connection.

Resistance: 3 x 750 Ohm.

Current: 3 x 2 A.

• N-INDT/1K. 1kVA Three-Phase inductances Module.

Parallel, series, star and delta connections..

Nominal Voltage: 400 VAC.

Inductances: 1,2H; 1,6H; 2H; 2,4H; 2,8H; 3,2H.

Currents: 0,65A; 0,5A; 0,45A; 0,35A; 0,30A; 0,25A.

• N-CON02. 3-Pole Contactor.

Supply Voltage Coil: 240 VAC.

Power terminals:

3 input power terminals.

3 output power terminals.

Auxiliary contacts:

3 normally open contacts (black).

3 normally closed contacts (red).

• N-REL09. Time Electronic Relay against Overcurrents (1.2 - 7 A).

Electronic thermal relay.

Overload protection.

Range 1.2 - 7 A.

Reset and test function.

1 NONC contact.

Temporization.

The AEL-SST-02 includes the following modules:

• N-TOPR. Time Overcurrent Protection Relay.

Nominal frequency: 50 to 60 Hz.

Thermal load capacity of current circuit:

Surge current (half-wave) 250x IN.

For 1s 100x IN

For 10s 30x IN

Continuous 4x IN

Output relay:

Switch-on current: 20 A.

Nominal current: 5 A.

Max. contact rating:

1250 VA/DC; 120W/DC, ohmic.

500VA/AC; 75W/DC, inductive.

Trip parameters:

I > 0.5-2x IN

I > 1-15x IN

tI > 0-100s

tI > 0-2.5s.

RS 485.

• N-BCT. Balancing Transformers Module.

Input current: 1 A.

Output current: 1 A.

Power voltage: 5 VA.

• N-PSM. Power Switch Module.

The voltage can be switch on and off manually or automatically.

Nominal voltage: 230/400V.

Frequency: 50/60Hz.

Control voltage: 24V.

Nominal operating current: 15 A.

Functions: 2 pushbutton switches and remote control for switch-off relay.

Contacts:

Three normally open contact.

One auxiliary normally open contact.

Optional module:

•N-AE1/1K. 1KVA Transmission Line Simulator Module.

Phase Resistance: 4 Ohm; 8 Ohm.

Phase Inductance: 125 mH 250 mH.

Phase Capacity:

Line-to-line: 2 x 100nF; 175nF.

Line-to-ground: 2 x 0,4µF; 0,8µF.

Maximum power: 1 kVA.

Nominal Voltage: 3 x 400V; 50/60Hz.

Maximum Current: 4,5 A.

•All necessary cables to realize the practical exercises are included.

The complete unit and optional PSV-SCADA includes as well:

Advanced Real-Time PSV-SCADA.

Open Control + Multicontrol + Real-Time Control.

Specialized EDIBON Control Software based on LabVIEW.

Projector and/or electronic whiteboard compatibility allows the unit to be explained and demonstrated to an entire class at one time.

Capable of doing applied research, real industrial simulation, training courses, etc.

Remote operation and control by the user and remote control for EDIBON technical support, are always included.

Totally safe, utilizing 4 safety systems (Mechanical, Electrical, Electronic & Software).

Designed and manufactured under several quality standards.

Optional ICAI software to create, edit and carry out practical exercises, tests, exams, calculations, etc.

Apart from monitoring user’s knowwledge and progress reached.

This unit has been designed for future expansion and integration. A common expansion is the EDIBON Scada-Net (ESN) System which enables multiple students to simultaneously operate many units in a network.

2 PSV-SCADA. Power Systems SCADA Viewer.

Compatible with the industry standards.

Flexible, open and multicontrol software, developed with actual windows graphic systems, acting simultaneously on all process parameters.

Management, processing, comparison and storage of data.

It allows the registration of the alarms state and the graphic representation in real time.

Open software, allowing the teacher to modify texts, instructions. Teacher’s and student’s passwords to facilitate the teacher’s control on the student, and allowing the access to different work levels.

This unit allows the 30 students of the classroom to visualize simultaneously all the results and the manipulation of the unit, during the process, by using a projector or an electronic whiteboard.

3 Cables and Accessories, for normal operation.

4 Manuals:

This unit is supplied with 7 manuals: Required Services, Assembly and Installation, Control Software, Starting-up, Safety, Maintenance & Practices Manuals.

*	References 1 to 4 are the main items: AEL-SST-02 + PSV-SCADA + Cables and Accessories + Manuals are included in the minimum supply for enabling  normal and full operation.

EXERCISES AND PRACTICAL POSSIBILITIES TO BE DONE WITH THE MAIN ITEMS

Some practical exercises possibilities with AEL-SST-01:

1.- Basic wiring with double busbar.

2.- Analyzing of double busbar connected to load.

3.- Busbar changeover without interruption

4.- Busbar operations with circuit breakers and disconnectors.

Some practical exercises possibilities with optional modules:

5.- Busbar coupling

6.- Voltage increases on open-circuit lines.

7.- Voltage drop as a function of line length.

8.- Voltage drop as a function of cos-phi.

9.- Capacitive and inductive power losses on a line as a function of U and I.

10.- Remote control operations in switching substations (with PSV-SCADA).

11.- Remote electrical measurements in several points of switching substation (with PSV-SCADA).

Additional practical exercises possibilities with AEL-SST-02:

12.- Phase shift on a line.

13.- Setting of the overcurrent relay.

14.- Double-pole short circuit.

15.- Single-pole short circuit.

16.- Triple-pole short circuit.

Other possibilities to be done with this Unit:

17.- Many students view results simultaneously.

To view all results in real time in the classroom by means of a projector or an electronic whiteboard.

18.- The Computer Control System with PSV-SCADA allows a real industrial simulation.

19.- This unit is totally safe as uses mechanical, electrical and electronic, and software safety devices.

20.- This unit can be used for doing applied research.

21.- This unit can be used for giving training courses to Industries even to other Technical Education Institutions.

- Several other exercises can be done and designed by the user.

Tender Specifications (for optional items)

a) Technical and Vocational Education configuration

5 AEL-SST-02/ICAI. Interactive Computer Aided Instruction Software System.

This complete software package consists of an Instructor Software (EDIBON Classroom Manager –ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft –ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

-ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc...so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

User Data Base Management.

Administration and assignment of Workgroup, Task and Training sessions.

Creation and Integration of Practical Exercises and Multimedia Resources.

Custom Design of Evaluation Methods.

Creation and assignment of Formulas & Equations.

Equation System Solver Engine.

Updatable Contents.

Report generation, User Progression Monitoring and Statistics.

-ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired technical knowledge.

Innovative features:

Student Log-In & Self-Registration.

Existing Tasks checking & Monitoring.

Default contents & scheduled tasks available to be used from the first session.

Practical Exercises accomplishment by following the Manual provided by EDIBON.

Evaluation Methods to prove your knowledge and progression.

Test self-correction.

Calculations computing and plotting.

Equation System Solver Engine.

User Monitoring Learning & Printable Reports.

Multimedia-Supported auxiliary resources.

b) Multipost Expansions options

6 Mini ESN. EDIBON Mini Scada-Net System.

EDIBON Mini Scada-Net System allows up to 30 students to work with a Teaching Unit in any laboratory, simultaneously.

The Mini ESN system consists of the adaptation of any EDIBON Computer Controlled Unit with SCADA integrated in a local network.

This system allows to view/control the unit remotely, from any computer integrated in the local net (in the classroom), through the main computer connected to the unit.

Main characteristics:

- It allows up to 30 students to work simultaneously with the EDIBON Computer Controlled Unit with SCADA, connected in a local net.

- Open Control + Multicontrol + Real Time Control + Multi Student Post.

- Instructor controls and explains to all students at the same time.

- Any user/student can work doing "real time" control/multicontrol and visualisation.

- Instructor can see in the computer what any user/student is doing in the unit.

- Continuous communication between the instructor and all the users/students connected.

Main advantages:

- It allows an easier and quicker understanding.

- This system allows you can save time and cost.

- Future expansions with more EDIBON Units.

The system basically will consist of:

This system is used with a Computer Controlled Unit.

- Instructor’s computer.

- Students’ computers.

- Local Network.

- Unit-Control Interface adaptation.

- Unit Software adaptation.

- Webcam.

- Mini ESN Software to control the whole system.

- Cables and accessories required for a normal operation.