TENDER SPECIFICATIONS

The WIT-ISE application includes the following elements:

• FP-STR. Assembly Frame with Safe Electrical Power Supply.

Aluminium structure:

Three aluminium struts.

Easy assembly of components via hammer head screws.

Possibility of simultaneous work of several students.

Four swiveling casters to facilitate the movement.

Dimensions:

Structure height: 1800 mm.

Useful working height: 1000 mm.

Width: 1500 mm.

Three-Phase connection plug.

Safe electric box:

Differential magnetothermal, 4 poles, 25A, 300mA AC 6KA.

Emergency stop mushroom (230/400V AC).

5-wire hose for connection to frame.

Signal lamp of voltage presence.

• FP-KIT-4. Wiring Installation Kit.

100 meters of grey wire of 1.5 mm².

100 meters of brown wire of 1.5 mm².

100 meters of black wire of 1.5 mm².

100 meters of green/yellow wire of de 1.5 mm².

25 meters of screened wire.

• FP-KIT-9. 400 VAC Contactors Kit.

Five three pole circuit breakers:

Nominal voltaje of power terminals: 400V AC.

Nominal voltaje of control terminals: 230V AC.

Nominal voltaje coil: 230V AC.

Auxiliary contacts: 3 NO / 2 NC.

• FP-KIT-10. Tubes and Fittings for Channeling Wiring Kit.

PVC electrical conduit for wiring installation, 20 m.

7 electrical boxes.

Cable guide.

20 PMA cable clamps 20 mm.

• FP-KIT-56. Industrial Elements Kit 1.

2 x Synchronism relay.

Current monitoring relay, 24V.

Red lighting indicator, 22 mm.

Green lighting indicator, 22 mm.

Commutator.

2 x Push buttons.

• FP-KIT-58. Fuses Kit.

10 x 2 A fuse.

10 x 10 A fuse.

Required Elements:

• CHER. Tool Box.

Crimper.

Tin.

Meter.

Insulation tape.

Heat shrink.

Voltage tester screwdriver.

Cross-head screwdriver and flat-head screwdriver.

Allen keys.

Soldering iron.

Rubber hammer.

Wire terminals and Connection terminals.

Flanges.

Wire cutter.

Screw Clamp Terminals.

Wire stripper.

Recommended elements:

• MED65. Digital Multimeter.

This module has a digital multimeter of about 3 ½ digits, with double-jack ending cables of about 4 mm to facilitate interconnections.

With this digital multimeter we will be able to measure:

Voltage.

Current.

Resistance.

Capacitors capacity.

Temperature.

• FP-KIT-10/B. Rigid Tube Kit.

20 m PVC Rigid Tube.

10 x Threaded connector.

5 x Elbows for conducts clamping.

• FP-KIT-5. Measuring Kit.

Clamp Meter:

Clamp for alternating/direct current measurements contactless.

The clamp can measure:

Current.

Voltage.

Resistance.

A voltage and continuity tester:

Voltage range: 12-690V AC.

Phases rotating detection in three-phase systems.

Polarity tester.

• EMT7-E. Asynchronous Three-Phase Motor of Squirrel Cage.

Nominal power: 370 W.

Nominal voltage: 3x 230/400V AC Δ/Y.

Frequency: 50/60 Hz

Number of poles: 2.

RPM: 2730 r.p.m.

Nominal current: 1,67/ 0,97 A.

• EMT9-E. Dahlander Three-Phase Motor.

Nominal power: 370 W.

Nominal voltage: 3x 400V AC.

Frequency: 50/60 Hz.

Number of poles: 4.

RPM: 1400/2800 r.p.m.

Nominal current: 1,2 / 1,55 A.

• EMT10-E. Asynchronous Three-Phase Motor with Two Independent Speeds.

Nominal power: 240/370 W.

Nominal voltage: 3x 400V AC.

Frequency: 50/60 Hz.

RPM: 900/1420 r.p.m.

Nominal current: 1,05 / 1,35 A.

• All necessary cables to realize the practical exercises are included.

Cables and Accessories, for normal operation.

Manuals:

This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

- Dimensions: 1500 x 400 x 1800 mm approx. (78.74 x 15.75 x 78.74 inches approx.) - Weight:  80 Kg approx. (220 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.-  Learning exercises about wiring techniques such as flat wiring, channel wiring and chamber wiring.

2.-  Configuration and commissioning of circuits incorporating these technologies.

3.-  Contactor control with circuit-breaker.

4.-  Contactor control with inching mode.

5.-  Contactor control with self-holding

6.-  Contactor control with two operating locations.

7.-  Inversion circuit with contactors.

8.-  Forced sequential connections with main current circuit

9.-  Interlocking of a contactor control.

10.-Contactor controls with a timing relay.

11.-Contactor circuits for motors with 2 speeds and 2 separate windings.

12.-Dahlander circuit with contactors.

13.-Star-delta circuit with contactors.

14.-Starter circuit for a slip-ring motor.

- Several other exercises can be done and designed by the user.

Optional

WIT-ISE/ICAI. Interactive Computer Aided Instruction Software System:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.