Since the impedance of a transmission line is proportional to its length, for distance measurement it is appropriate to use a relay capable of measuring the impedance of a line up to a predetermined point (the reach point). Such a relay is described as a distance relay and is designed to operate only for faults occurring between the relay location and the selected reach point, thus giving discrimination for faults that may occur in different line sections.

The basic principle of distance protection involves the division of the voltage at the relaying point by the measured current. The apparent impedance so calculated is compared with the reach point impedance. If the measured impedance is less than the reach point impedance, it is assumed that a fault exists on the line between the relay and the reach point.

The AEL-TPT-07 application has been developed as a complement to AEL-TPT-01 in order to study how protect a system with high speed distance protection relay.

Tender Specifications (for main items)

1 AEL-TPT-07. Unit.

The trainer requires the following modules:

•N-VPS01. AC 3PH Variable Power Supply.

ON-OFF removable key.

Input Voltage: 400 VAC, 3PH+N

Output Voltage:

0…450 VAC

0…240 VDC

Maximum current: 2A

Frequency: 50/60 Hz

Wheel to regulate the voltage.

Three lamps.

Measurement commutators:

One commutator to measure the voltage:

6 positions: L1-N ; L2-N ; L3-N ; L1-L2 ; L1-L3 ; L2-L3

One commutator to measure the current:

3 positions: L1, L2, L3

Three-Phase supply hose with IP44 3PN+E 32A 400V connecting plug.

Differential magnetothermal, 4 poles, 25A, 30mA AC 6KA

•N-MED65. Digital Multimeter.

This module has a digital multimeter of about 3 ½ digits, with double-jack ending cables of about 4 mm to facilitate interconnections.

With this digital multimeter we will be able to measure:

Voltage.

Current.

Resistance.

Capacitors capacity.

Temperature.

•N-REVT/1K. 1kW Three-Phase Resistors Module.

Parallel, series, star and delta connection.

Resistance: 3 x 750 Ohm

Current: 3 x 2 A

•N-EALD. Network Analyzer Unit with Computer Data Acquisition (2 units).

ON-OFF switch.

Supply voltage: 400 VAC

Input terminals: Input connection with the measurement point.

Output terminals: Output connection with the measurement point.

Digital outputs: Three digital outputs are used for pulses or alarms, or for combining both.

RS-485 Communication port.

Fuses: 3x10 A

Network Analyzer Display. It shows:

Active, reactive and apparent power.

Active, reactive and apparent energies.

Lines and phase currents.

Line and phase voltages.

Frequencies.

Power Factor.

•N-AE1/1K. 1KVA Transmission Line Simulator Module.

Phase Resistance: 4 Ohm; 8 Ohm

Phase Inductance: 125mH 250mH

Phase Capacity:

Line-to-line: 2 x 100nF; 175nF

Line-to-ground: 2 x 0,4μF; 0,8μF

Maximum power: 1kVA

Nominal Voltage: 3 x 400V; 50/60Hz

Maximum Current: 4,5A

•N-PSM. Power Switch Module.

The voltage can be switch on and off manually or automatically.

Nominal voltage: 230/400V

Frequency: 50/60Hz

Control voltage: 24VAC

Nominal operating current: 15A

Functions: 2 pushbutton switches and remote control for switch-off relay.

Contacts:

Three normally open contact.

One auxiliary normally open contact.

•N-TOPR. Time Overcurrent Protection Relay.

Nominal frequency: 50 to 60 Hz.

Thermal load capacity of current circuit:

Surge current (half-wave) 250x IN.

For 1s 100x IN.

For 10s 30x IN.

Continuous 4x IN.

Output relay:

Switch-on current: 20A.

Nominal current: 5A.

Max. contact rating:

1250 VA/DC; 120W/DC, ohmic.

500VA/AC; 75W/DC, inductive.

Trip parameters:

I > 0.5-2x IN.

I > 1-15x IN.

tI > 0-100s.

tI > 0-2.5s.

RS 485.

•N-RELT. Relay test module.

Power supply: 110-230V AC.

Frequency: 50/60Hz.

Start/stop pushbutton.

Display.

Operation pushbuttons:

Switch-on (it can be remote controlled).

Switch-off (it can be remote controlled).

Timer ON (relay test).

Contacts:

Voltage: 0…400 VAC/VDC.

Current: 0…20 A AC/DC.

3 operating contacts.

2 auxiliary contacts.

•N-CON02. 3-Pole Contactor.

Supply Voltage Coil: 240 VAC

Power terminals:

3 input power terminals.

3 output power terminals.

Auxiliary contacts:

3 normally open contacts (black).

3 normally closed contacts (red).

•N-REL09. Time Electronic Relay against Overcurrents (1.2 - 7 A).

Electronic thermal relay.

Overload protection.

Range 1.2 - 7 A.

Reset and test function.

1 NONC contact.

Temporization.

The trainer AEL-TPT-07 includes the following modules:

•N-HSDPR. High Speed Distance Protection Relay.

Nominal voltage: 80...125V.

Nominal frequency: 50Hz/60Hz.

Nominal current: 1A.

Distance protection (ANSI 21/21N).

Six distance stages.

Polygon- or circular characteristic.

Teleprotection for distance protection (ANSI 85-21).

Overvoltage/undervoltage protection (ANSI 59, 27).

Over/underfrequency protection (ANSI 81).

Thermal overload protection (ANSI 49).

Backup overcurrent protection (ANSI 50, 50N, 51, 51N).

STUB-bus overcurrent stage (ANSI 50-STUB).

Switch-onto-fault protection (ANSI 50HS).

Trip circuit supervision (ANSI 74TC).

CLOSE command interlocking (Lockout, ANSI 86).

Inputs and Outputs:

5 Indication inputs.

4 command outputs.

5 fast relays.

Measuring:

Current.

Voltage.

Power Factor.

Active and reactive power.

•N-TRTC. Three-Phase Current Transformer.

Input current: 2.5A.

Output current: 1A.

Nominal Power: 1VA.

Accuracy: class 1.

Overcurrent factor: FS 10.

•N-TRTV. Three-Phase Voltage Transformer.

Input voltage max: 3x 230VAC.

Output voltage max: 3x 63 VAC.

Nominal Power: 1VA.

Vector group: YY0.

•N-AE1/1K. 1KVA Transmission Line Simulator Module.

Phase Resistance: 4 Ohm; 8 Ohm.

Phase Inductance: 125mH 250mH.

Phase Capacity:

Line-to-line: 2 x 100nF; 175nF

Line-to-ground: 2 x 0,4µF; 0,8µF

Maximum power: 1kVA

Nominal Voltage: 3x 400V; 50/60Hz

Maximum Current: 2 A

•All necessary cables to realize the practical exercises are included.

The complete unit includes as well:

Advanced Real-Time SCADA.

Open Control + Multicontrol + Real-Time Control.

Specialized EDIBON Control Software based on LabVIEW.

Projector and/or electronic whiteboard compatibility allows the unit to be explained and demonstrated to an entire class at one time.

Capable of doing applied research, real industrial simulation, training courses, etc.

Remote operation and control by the user and remote control for EDIBON technical support, are always included.

Totally safe, utilizing 4 safety systems (Mechanical, Electrical, Electronic & Software).

Designed and manufactured under several quality standards.

Optional ICAI software to create, edit and carry out practical exercises, tests, exams, calculations, etc.

Apart from monitoring user’s knowwledge and progress reached.

This unit has been designed for future expansion and integration. A common expansion is the EDIBON Scada-Net (ESN) System which enables multiple students to simultaneously operate many units in a network.

2 AEL-TPT-07/CCSOF. Computer Control + Data Acquisition + Data Management Software: .

The three softwares are part of the SCADA system.

Compatible with the industry standards.

Flexible, open and multicontrol software, developed with actual windows graphic systems, acting simultaneously on all process parameters.

Management, processing, comparison and storage of data.

It allows the registration of the alarms state and the graphic representation in real time.

Open software, allowing the teacher to modify texts, instructions. Teacher’s and student’s passwords to facilitate the teacher’s control on the student, and allowing the access to different work levels.

This unit allows the 30 students of the classroom to visualize simultaneously all the results and the manipulation of the unit, during the process, by using a projector or an electronic whiteboard.

3 Cables and Accessories, for normal operation.

4 Manuals:

This unit is supplied with 7 manuals: Required Services, Assembly and Installation, Control Software, Starting-up, Safety, Maintenance & Practices Manuals.

*	References 1 to 4 are the main items: AEL-TPT-07 + AEL-TPT-07/CCSOF + Cables and Accessories + Manuals are included in the minimum supply for enabling  normal and full operation.

EXERCISES AND PRACTICAL POSSIBILITIES TO BE DONE WITH THE MAIN ITEMS

Some practical exercises possibilites of basic equipment (AEL-TPT-01):

1.- Configuration of overcurrent time protection relay.

2.- Parameterization of overcurrent time protection relay.

3.- Commissioning of overcurrent time protection relay.

4.- Configuration of different protection thresholds.

5.- Generation of different current levels for tripping operations.

6.- Analysis of different wave forms in real time.

7.- Determining rise and fall time.

8.- Determining the reset ratio.

9.- Setting and testing various characteristics.

10.- Checking circuit breaker´s release behavior in the event of a failure.

Some practical exercises possibilities with the specific modules of AEL-TPT-07:

11.- Configuration of distance protection relay.

12.- Parameterization of distance protection relay.

13.- Commissioning of distance protection relay.

14.- Configuration of distance protection relay thresholds.

15.- Setting and testing Distance protection characteristic.

16.- Setting and testing Overcurrent time protection characteristic.

17.- Setting and testing Voltage protection characteristic.

Other possibilities to be done with this Unit:

18.- Many students view results simultaneously.

To view all results in real time in the classroom by means of a projector or an electronic whiteboard.

19.- The Computer Control System with SCADA allows a real industrial simulation.

20.- This unit is totally safe as uses mechanical, electrical and electronic, and software safety devices.

21.- This unit can be used for doing applied research.

22.- This unit can be used for giving training courses to Industries even to other Technical Education Institutions.

- Several other exercises can be done and designed by the user.

Tender Specifications (for optional items)

a) Technical and Vocational Education configuration

5 AEL-TPT-07/ICAI. Interactive Computer Aided Instruction Software System.

This complete software package consists of an Instructor Software (EDIBON Classroom Manager –ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft –ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

-ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc...so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

User Data Base Management.

Administration and assignment of Workgroup, Task and Training sessions.

Creation and Integration of Practical Exercises and Multimedia Resources.

Custom Design of Evaluation Methods.

Creation and assignment of Formulas & Equations.

Equation System Solver Engine.

Updatable Contents.

Report generation, User Progression Monitoring and Statistics.

-ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired technical knowledge.

Innovative features:

Student Log-In & Self-Registration.

Existing Tasks checking & Monitoring.

Default contents & scheduled tasks available to be used from the first session.

Practical Exercises accomplishment by following the Manual provided by EDIBON.

Evaluation Methods to prove your knowledge and progression.

Test self-correction.

Calculations computing and plotting.

Equation System Solver Engine.

User Monitoring Learning & Printable Reports.

Multimedia-Supported auxiliary resources.

b) Multipost Expansions options

6 Mini ESN. EDIBON Mini Scada-Net System.

EDIBON Mini Scada-Net System allows up to 30 students to work with a Teaching Unit in any laboratory, simultaneously.

The Mini ESN system consists of the adaptation of any EDIBON Computer Controlled Unit with SCADA integrated in a local network.

This system allows to view/control the unit remotely, from any computer integrated in the local net (in the classroom), through the main computer connected to the unit.

Main characteristics:

- It allows up to 30 students to work simultaneously with the EDIBON Computer Controlled Unit with SCADA, connected in a local net.

- Open Control + Multicontrol + Real Time Control + Multi Student Post.

- Instructor controls and explains to all students at the same time.

- Any user/student can work doing "real time" control/multicontrol and visualisation.

- Instructor can see in the computer what any user/student is doing in the unit.

- Continuous communication between the instructor and all the users/students connected.

Main advantages:

- It allows an easier and quicker understanding.

- This system allows you can save time and cost.

- Future expansions with more EDIBON Units.

The system basically will consist of:

This system is used with a Computer Controlled Unit.

- Instructor’s computer.

- Students’ computers.

- Local Network.

- Unit-Control Interface adaptation.

- Unit Software adaptation.

- Webcam.

- Mini ESN Software to control the whole system.

- Cables and accessories required for a normal operation.