TENDER SPECIFICATIONS

Designed to work with EDIBON units.

Single-phase motor with 340 W.

Low noise level with 40 dB of maximum.

Boiler capacity: 9 l.

Maximum pressure: 8 bar or 0.8 MPa.

Maximum air flow: 50 l/min.

Safety relief valve.

Start/stop switch.

Double scale manometer psi and bar with a range from 0 to 12 bar to measure the air pressure before the air filter-regulator.

Air Filter-Regulator (FR):

Double scale manometer psi and bar with a range from 0 to 12 bar to measure the air pressure after the air filter regulator.

Air filter with drain valve.

Air pressure regulator from 0 to 8 bar.

6 mm quick release connector for pneumatic flexible tubes.

Oil level indicator.

The oil necessary to work is included.

The connectors required to work with EDIBON pneumatic units are included.

- Dimensions: 400 x 350 x 350 mm approx. (15.74 x 13.77 x 13.77 inches approx.). -Weight: 15 Kg approx. (33 pounds approx.).