Tender Specifications

- FACO. Power Supply:

Fixed outputs DC: + 5 V, ± 12 V, 1 A.

Variable outputs DC: ± 12 V, 0.5 A.

AC output: 12 V or 24 V.

Connection to the PLC-BS modules through 25 pin Sub-D connectors.

LED’s voltage indicators.

Robust construction.

- BS6. Liquid level Test Module:

Painted steel box. Connection diagrams for each transducer are represented graphically.

Water tanks.

Capacitive level sensor:

Level sensor immersed in the tank. Power supply: 12 – 35 Vdc. Output: 4 – 20 mAdc.

Level sensor by pressure:

It is a differential pressure sensor that measures the pressure practiced by the water compared to the atmospheric pressure to measure the water level. Pressure range: 0 – 1 psi. Sensibility: 16.7 mV/psi. Excitation voltage: 10 – 16 Vdc.

Float level switches with potentiometer:

It is a potentiometer fixed to a float arm that will vary its position depending to the water level. This system complements itself with two end and beginning path switches respectively. Switching voltage contacts: 250 Vac/125 Vdc.

Conductivity level sensor:

This sensor works with two electrodes immersed in one of the tanks. As the water level rises and covers the electrodes its resistance will decrease until it arrives to KΩ unit values; as long as the water does not touch the electrodes, the resistance between them will be very big and will behave like an open circuit.

Magnetic float level sensor:

Sensor formed by a small float that has inside a magnetic element, the float base has a Hall effect element that detects when the float has gone up due to the effect of the water. Switching voltage: 240 Vac, 120 Vdc.

Optical level sensor:

It is a photodiode and phototransistor, which in presence of water changes its refraction properties and make the output state approximately change from 3 Vdc  – 0 Vdc. Power supply: 5 Vdc. Load current: 20 mA max. at 125 ºC.

Two pumps:

The volume supplied by these pumps can be regulated varying the dc voltage value with which they are supplied. Power supply: 12 Vdc (max. voltage). Nominal volume: 1 l/min. Nominal current: 1 A DC.

Feedback sensor for PLC control:

Signal conditioning circuit of the tachometer.

Voltage output range: 0 – 10 Vdc.

Cables and Accessories, for normal operation.

Manuals: This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manual.

Each module may operate independently of another.

Dimensions: 405 x 300 x 380 mm approx. (15.94 x 11.81 x 14.96 inches approx.)  Weight: 10 Kg approx. (22.04 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.-  To use a capacitive sensor to measure the liquid level in the tank.

2.-  To use the differential pressure sensor as an element to determine the water level in a tank.

3.-  To use a changeable resistance fixed to a float system as a liquid level measurement element.

4.-  Conduction Sensor. Use of a sensor made up of to steel electrodes to measure the water level of a tank.

5.-  Magnetic float level sensor. It detects a precise liquid level in the tank with a magnetic contact.

6.-  Control of the module left tank liquid level using an optical level sensor.

7.-  Using the analog inputs and outputs of the PLC to read and write analog signals.

8.-  Read the tachometer signal with the PLC.

9.-  Manual control of the actuator through the PLC.

10.-PID control of the angular speed value through the PLC.

11.-Observe the effects of the angular speed control to changes in the PID parameters.

Optional

PLC-BS6/ICAI. Interactive Computer Aided Instruction Software System:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.