Tender Specifications

Bench-top unit with adjustable legs.

Anodized aluminum structure and panels in painted steel.

The MELH unit mainly consists of:

2 springs with the following features:

Length: 100 mm.

External diameter: 18 mm.

Wire diameter: 1.2 mm.

Spring with the following features:

Length: 100 mm.

External diameter: 18 mm.

Wire diameter: 1.1 mm.

These springs can be configured in series and in parallel.

2 rules to measure deformations, graduations: 1mm.

In order to carry out some of the practices with MELH unit, a "B" Type Set of weights is required.

Manuals: This unit is supplied with the following manuals: Required services, Assembly and Installation, Starting-up, Safety, Maintenance and Practices manual.

Dimensions:  300 x 450 x 1100 mm. approx.  (11.81 x 17.71 x 43.31 inches approx.).   Weight: 10 Kg. approx. (22 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Demonstration of Hooke's law.

2.- Research on the proportionality of force applied and elongation of the spring.

3.- Determination of the spring constant.

4.- Study of the arrangement in series of two tension springs.

5.- Study of the arrangement in parallel of two tension springs.

6.- Research on the influence of the spring constant on the oscillation frequency of a mass-spring system.

Optional

MELH/ICAI. Interactive Computer Aided Instruction Software System:

this complete software package consists of an Instructor Software (EDIBON Classroom Manager -ECM) totally integrated with the Student Software (EDIBON Student Labsoft -ESL). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

-ECM. EDIBON Classroom Manager (Instructor Software).

ECM is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc...so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

User Data Base Management.

Administration and assignment of Workgroup, Task and Training sessions.

Creation and Integration of Practical Exercises and Multimedia Resources.

Custom Design of Evaluation Methods.

Creation and assignment of Formulas & Equations.

Equation System Solver Engine.

Updatable Contents.

Report generation, User Progression Monitoring and Statistics.

-ESL. EDIBON Student Labsoft (Student Software).

ESL is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired technical knowledge.

Innovative features:

Student Log-In & Self-Registration.

Existing Tasks checking & Monitoring.

Default contents & scheduled tasks available to be used from the first session.

Practical Exercises accomplishment by following the Manual provided by EDIBON.

Evaluation Methods to prove your knowledge and progression.

Test self-correction.

Calculations computing and plotting.

Equation System Solver Engine.

User Monitoring Learning & Printable Reports.

Multimedia-Supported auxiliary resources.

BDAS. Basic Data Acquisition System and Sensors.

For being used with mechanical modules.

BDAS is designed to monitor the measurements of each mechanical module from a computer (PC).