Tender Specifications

Bench-top unit with adjustable legs.

Anodized aluminum frame and panels made of painted steel.

The "MFBS" unit mainly consists of:

Bars made of stainless steel:

Three bars of 385 mm long, diameter: 8 mm.

Bar of 575 mm long, diameter: 8 mm.

Bar of 720 mm long, diameter: 8 mm.

Each bar has a force meter attached to it that consists of:

Set of flat bars made of aluminum, dimensions: 90 x 25 x 1 mm.

Four nuts to adjust the length of the bars.

Dial gauge, measuring range: 0 – 10 mm, graduations: 0.01 mm.

Three node discs, diameter: 95 mm, with different positions to vary the angle between bars.

Two of them support the bars of the structure and this structure to the base of the unit (one is a fixed support and the other is a movable support).

The other (upper node disc) supports the bars of the structure and is used to hang different masses.

Set of masses consisting of:

Mass of 500 g (5 N).

Mass of 1 Kg (10 N).

Two masses of 2 Kg (20 N).

Hook to hang the masses.

Manuals: This unit is supplied with the following manuals: Required services, Assembly and Installation, Starting-up, Safety, Maintenance and Practices manual.

Dimensions: 1000 x 300 x 500 mm approx. (39.37 x 11.81 x 19.68 inches approx.)  Weight: 15 Kg approx. (33 pounds approx)

EXERCISES AND PRACTICAL POSSIBILITIES

1.-  Measurement of efforts on the bars of a simple bar structure.

2.-  Measurement of efforts on the bars, in different configurations of a simple bar structure.

3.-  Theoretical calculation of efforts in the bars, in different configurations of a simple bar structure, using the method of joints.

4.-  Theoretical calculation of efforts in the bars, in different configurations of a simple bar structure, using the graphical method.

5.-  Comparison of the experimental method with the theoretical methods: method of joints and graphical method.

Optional

MFBS/ICAI. Interactive Computer Aided Instruction Software System:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.

BDAS. Basic Data Acquisition System and Sensors:

For being used with mechanical modules.

BDAS is designed to monitor the measurements of each mechanical module from a computer.