TENDER SPECIFICATIONS

The MCC unit, Gearbox, is mounted on a frame made of aluminum profiles and panel made of painted steel.

The gears and shafts are made of stainless steel.

The number of gears and the teeth of the gears is:

2 gears with 20 teeth each.

3 gears with 30 teeth each.

2 gears with 40 teeth each.

1 gear with 16 teeth.

The unit includes two pulleys, which are made of aluminum.

The shafts of the unit are mounted on ball bearings. Thus, inertia will be low and losses due to friction will decrease, allowing an easier execution of the practical exercises.

To perform the practical exercises the unit can be either placed on a table or hung from a wall, allowing a more comfortable performance of the practical exercises with the weights or masses.

In order to carry out some practices with MCC unit, 2 sets of weights “B type” are required. (See “Required Accessories (Not Included)” section).

Manuals: This unit is supplied with the following manuals: Required services, Assembly and Installation, Starting-up, Security, Maintenance and Practices manual.

- Dimensions: 580 x 350 x 500 mm. approx.  (22.83 x 13.78 x 19.68 inches approx.).  - Weight: 17 Kg. approx.   (37.47 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Study of mechanism that represents a gearbox.

2.- Measurement of the angular displacement at the input and output of the gearbox and comparison with the theoretical values.

3.- Determination of the minimum effort required at the input shaft to lift a series of loads.

4.- Calculation of the efficiency for different ratios depending on the input torque.

5.- Plot of characteristic curves.

6.- Determination of the torque and speed ratio for different gears.

Optional

MCC/ICAI. Interactive Computer Aided Instruction Software System:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.