Tender Specifications

Experimental unit for the training of apprentices in the refrigeration area.

Anodized aluminum frame and panels made of painted steel.

The unit includes wheels to facilitate its mobility.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Air conditioner:

Power consumption: 1.5 kW approx.

Cooling capacity: 3 kW approx.

Termal power: 3 kW approx.

Maximum air flow (inner unit): 420 m³/h approx.

Dehumidification (inner unit): 0.8  l/h approx.

Timer: 24 h.

Battery-operated remote control.

Operation modes: heating, cooling, dehumidifying, ventilating, service mode and automatic mode.

Additional functions configurable with the remote control: sleep, swing and timer.

Two manometers to measure the low pressure and high pressure of the compressor. Range: -1 to 40 bar.

The unit has been designed to be used with environmental friendly CFC-free refrigerant.

Cables and accessories, for normal operation.

Manuals: This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

Dimensions: 980 x 980 x 1450 mm. approx. (38.58 x 38.58 x 57.08 inches approx.)  Weight: 75 Kg. approx. (165.34 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.-  Study of the assembly and operation of a split system air conditioner.

2.-  Study of six different operation modes of the air conditioning system:

Cooling.

Heating.

Dehumidifying.

Ventilating.

Automatic service.

Service mode (test of cooling function).

3.-  Study of the three additional functions:

“SLEEP” night function.

“SWING” function.

24h switching on/off timer.

4.-  Study of the remote control functions.

Optional

TACS/ICAI. Interactive Computer Aided Instruction Software System:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc...so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.