Tender Specifications

Anodized aluminum frame and panels made of painted steel.

Diagram in the front panel with distribution of the elements similar to the real one.

It includes wheels to facilitate its mobility.

Eleven transparent toilet tanks with solenoid valves controlled by the user:

Six tanks of 10  l.

Four tanks of 20  l.

Tank of 9  l (toilet).

Two toilets with solenoid valves controlled by the user.

Toilet with tank.

Toilet with pressure flush.

All the pipes are made of PMMA to allow the visualization of the process.

Measuring instruments:

Six tube manometers: 1500 mm WC to measure pressure along the downpipe.

Manometer with range: 0.4 – 4  l/h.

Pump:

Power: 0.75 CV.

Flow: 250  l/min.

Head: up to 100 m.

Collection tank:

Volume: 300  l approx.

Electronic console:

Metallic box.

Solenoid valves switches.

Cables and Accessories, for normal operation.

Manuals: This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

Dimensions: Unit: 6000 x 800 x 4000 mm approx. (236.22 x 31.49 x 157.48 inches approx.)  Weight: 1200 Kg approx. (2645 pounds approx.)

Electronic console: 300 x 230 x 135 mm approx. (11.81 x 9.05 x 5.31 inches approx.)  Weight: 2 Kg approx. (4.4 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.-  Study of flow and pressure in bypass pipes.

2.-  Study of flow and pressure in sanitary valves and fittings.

3.-  Study of different drain pipes.

4.-  Study of an incorrect flow behavior with an error in the pipe sizing.

5.-  Study of incorrect flow behavior with defective aeration of the pipes.

6.-  Study of the pressure curve in the downpipe.

7.-  Study of the flow at a pipe offset.

8.-  Study of the behavior and the effect of suction at the junctions.

Optional

TSID/ICAI. Interactive Computer Aided Instruction Software System:

This complete software package consists of an Instructor Software (INS/SOF) totally integrated with the Student Software (TSID/SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.