TENDER SPECIFICATIONS

The Radar Trainer “ERA” consists of:

Interface unit:

It contains the display and the radar control system.

Display unit:

High definition 6” rectangular monochrome LCD display.

Display resolution: 240 x 320 pixels.

Markers: Heading Line, Bearing Scale, Range Rings, Variable Range Marker (VRM), Electronic Bearing Line (EBL), Tuning Bar, Cursor, Parallel Cursor, Alarm Zone, Waypoint Mark*, North Mark*.

Alphanumeric indications: Range, Range Ring Interval, Display Mode (HU), Interference Rejection (IR), Variable Range Marker (VRM), Electronic Bearing Line (EBL), Stand-By (ST-BY), Guard Alarm (G (IN), G (OUT), UP RANGE), Echo Stretch (ES), Range and Bearing to Cursor, Bearing or L/L Position, Echo Tailing (TRAIL), Trailing Time, Trailing Elapsed Time, Watchman (WATCHMAN), Zoomed Display (ZOOM), Navigation Data*, Heading* (HDC).

*:external data required.

Battery system:

Battery charger (included inside de interface unit).

Battery: nominal voltage: 12Vdc. capacity: 24AH.

Connector to the GPS module.

Connector to the Radar Antenna.

Radar Antenna:

Outdoor operation.

Directional antenna: Micro-strip patch array antenna.

Horizontal polarization.

Frequency 9.4 GHz (X-band).

Antenna rotation speed 24/31/41 rpm nominal (auto-select according to range).

Horizontal Beamwidth less than 6.2º. Vertical Beamwidth 25º.

Connection cable to the interface unit.

GPS module:

High contrast display with gray scale. Display resolution: 120x160 pixels.

GPS Accuracy: 15 m. DGPS (WAAS) Accuracy: 3 m.

Connection cable to the interface unit.

Extendable tripod stand:

Telescopic tripod stand, of anodized aluminum, to hold the radar antenna at the appropriate height. It includes a bull's eye level indicator. Operation range: 1.20 m – 3.05 m. Maximum supported weight: 7.5 kg.

Mobile cart:

Mobile cart to hold the elements of the ERA trainer. It incorporates wheels (strong tires) to facilitate its mobility and the operation in uneven grounds.

Cables and Accessories, for normal operation.

Manuals:

This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

Dimensions: Unit: 600 x 900 x 1700 mm. approx. (23.62 x 35.43 x 66.93 inches approx.).     Weight: 50 Kg. approx. (110 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.-	Familiarization with the trainer.

2.-	Establish communication with GPS module.

3.-	Configuration of the radar range.

4.-	Sensitivity adjustment.

5.-	Range measurement.

6.-	Bearing measurement.

7.-	Setting of the suppressing interference function.

8.-	Setting of the suppressing rain clutter function.

9.-	Analysis of the guard alarm function and establish a guard zone.

10.-Analysis of the watchman function.