TENDER SPECIFICATIONS

The AEL-AD9A application includes the following elements:

• N-ALI02. Domestic Main Power Supply.

Supply voltage (Single-Phase): 230 VAC, PH+N+G

ON-OFF removable key.

Output Voltage Connections:

Two Single-Phase: 230 VAC

Single-Phase supply hose connecting plug.

Differential magnetothermal, 2 poles, 25 A, 30 mA AC 6 KA.

• N-TIM01. Bell 70 dB Module (2 units).

Nominal voltage: 230 VAC.

Frequency: 50 Hz.

Sound pressure level: 70 dB

Ground terminal.

• N-SEL09. Double Luminous Signaling red-green Module (2 units).

Nominal voltage: 230 VAC.

Red and green luminous warning.

Power of luminous warning: 5 W.

Terminals:

Two Single-Phase terminals (PH+N) to connect the electrical warning.

Ground terminal.

• N-MED76. Thermostat for Heating.

Supply voltage: 230 VAC (PH+N).

Nominal current: 10 A.

Wheel to change the temperature:

5ºC to 30º regulation.

Accuracy: 0.25ºC.

Contacts:

One Normally Close Contact (NC).

Ground terminal.

• N-MED77. Thermostat for Heating and Refrigeration.

Supply voltage: 230 VAC (PH+N).

Nominal current: 10 A.

Wheel to change the temperature:

5ºC to 30º regulation.

Accuracy: 0.25ºC.

Contacts:

One Normally Open Contact (NO).

One Normally Close Contact (NC).

Common point of normally close/open contacts.

Ground terminal.

• All necessary cables to realize the practical exercises are included.

Cables and Accessories, for normal operation.

Manuals:

This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

- Dimensions: 640 x 320 x 670 mm approx. (25.19 x 12.59 x 26.37 inches approx.) - Weight:  60 Kg approx. (132 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.-   Identification of the elements of the main power supply domestic.

2.-   Checking the domestic main power supply.

3.-   Checking relays.

4.-   Checking the thermostat for heating and tests.

5.-   Checking the thermostat for heating and refrigeration, and tests.

6.-   Test with several temperatures and green light.

7.-   Test with several temperatures and red light.

8.-   Test with several temperatures and the bell.

9.-   Test with several temperatures, red light and the bell.

10.- Test with several temperatures, green light and the bell.

Optional

AEL-AD9A/ICAI. Interactive Computer Aided Instruction Software System:

This complete software package consists of an Instructor Software (INS/SOF) totally integrated with the Student Software (AEL-AD9A/SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.