TENDER SPECIFICATIONS

The application includes the following modules:

• N-ALI02. Domestic Main Power Supply.

Voltage supply (Single-Phase): 230 VAC, PH+N+G.

ON-OFF removable key.

Output Voltage Connections:

Two Single-Phase: 230 VAC.

Single-Phase supply wire connecting plug.

Differential magnetothermal, 2 poles, 25 A, 30 mA AC 6 KA.

• N-ALI03. AC Auxiliary Power Supply.

Voltage supply (Single-Phase): 230 VAC PH+N+G.

Output voltage:

Single-Phase 24 VAC/12 VAC.

24 VDC.

0 - 24 VDC through potentiometer.

• N-LAM16 Halogen Lamp.

Voltage supply, 230 VAC.

Power, 60 W.

• N-SEL03 Three Pilot-Lights.

Voltage supply, 24 VAC.

Colors: red, green and yellow.

• N-SEN04. Inductive Proximity Sensor type PNP.

Voltage supply, 24 VDC.

Output voltage, 0-24 VDC.

It detects metallic bodies.

1 NO contact.

PNP output.

15 mm maximum detection.

Status and detection leds.

Input and output terminals.

•  N-SEN14. Capacitive Proximity Sensor.

Voltage supply, 24 VDC.

Output voltage, 0-24 VDC.

It detects metallic and non metallic bodies.

10 mm maximum detection.

Status and detection leds.

Fuse 250 mA.

Input and output terminals.

• N-SEN29. Inductive Proximity Sensor.

Voltage supply, 24 VAC.

Output voltage, 0-24 VDC.

It detects metallic bodies.

10 mm maximum detection.

Fuse 250 mA.

Input and output terminals.

• N-SEN26 Presence and Motion Sensor (Wall).

Voltage supply, 230 VAC.

One output.

0 to 180º detection.

Maximum distance detection 12 m.

Timing 10 sec.-15 min.

Luminosity regulation 2-120 lux.

Terminals.

• All necessary cables to realize the practical exercises are included.

Cables and Accessories, for normal operation.

Manuals:

This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

- Dimensions: 640 x 320 x 670 mm approx. (25.19 x 12.59 x 26.37 inches approx.) - Weight: 60 Kg approx. (132 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.-  Checking and measuring on the main power supply.

2.-  Checking the working of the auxiliary power supply.

3.-  Checking the operation of the inductive sensor type PNP.

4.-  Checking the operation of the capacitive sensor.

5.-  Checking the operation of the AC inductive sensor.

6.-  Body detection tests with the different types of sensors and materials.

7.-  Body movement detection tests with the presence sensor.

8.-  Real applications of a detection and position control system.

Optional

AEL-AD15A/ICAI. Interactive Computer Aided Instruction Software System:

This complete software package consists of an Instructor Software (INS/SOF) totally integrated with the Student Software (AEL-AD15A/SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.