TENDER SPECIFICATIONS

The application includes the following elements:

• TPT01. Three-Phase Transformer 01.

Three-phase transformer.

Nominal supply voltage: 400 VAC (3PH).

Nominal output voltage: 3 x 230 VAC (3PH+N).

Nominal power: 1 kVA.

• CHER. Toolbox.

It contains the following tools:

Allen key.

Cross-head screwdriver.

Flat-head screwdriver.

Rubber hammer.

Insulation tape.

Soldering iron.

Tin.

Heat shrink.

Additional and recommended modules and elements:

• N-VPS01. AC 3PH Vaiable Power Supply.

ON-OFF removable key.

Input Voltage: 400 VAC, 3 PH + N.

Output Voltage:

0…450 VAC.

0…240 VDC.

Maximum current: 2 A.

Frequency: 50/60 Hz.

Wheel to regulate the voltage.

Three lamps.

Measurement commutators:

One commutator to measure the voltage:

6 positions: L1-N ; L2-N ; L3-N ; L1-L2 ; L1-L3 ; L2-L3

One commutator to measure the current:

3 positions: I1, I2, I3.

Three-Phase supply hose with IP44 3PN+E 32 A 400 V connecting plug.

Differential magnetothermal, 4 poles, 25 A, 300 mA AC 6 KA.

• MWMT. Manual Winding for Motors and Transformers.

Mechanical flywell.

Winding templates.

Spin counter.

• MED65. Digital Multimeter.

Digital multimeter of about 3 ½ digits, with double-jack ending cables of about 4 mm to facilitate interconnections.

With this digital multimeter we will be able to measure:

Voltage.

Current.

Resistance.

Capacitors capacity.

Temperature.

• All necessary cables to realize the practical exercises are included.

Cables and Accessories, for normal operation.

Manuals:

This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

AEL-TPTC: - Dimensions: 640 x 320 x 670 mm. approx. (25.19 x 12.59 x 26.27 inches approx.) - Weight: 35 Kg. approx. (77 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Step by step construction of a three-phase transformer.

2.- Study of different parts of a three-phase transformer.

Additional practical possibilities (with the additional and recommended modules and elements):

3.- Measurement of the transformer coils.

4.- Testing of the three-phase transformer after the construction.

5.- Measurement of the different voltages in the primary and secondary wounds.

6.- Designing different types of coils using a winding machine.

Optional

AEL-TPTC/ICAI. Interactive Computer Aided Instruction Software System:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc...so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.