TENDER SPECIFICATIONS

The AEL-RPC includes the following elements:

• N-ALI01. Industrial Power Supply.

Supply voltage: 400 VAC, 3PH+N+G.

ON-OFF removable key.

Output voltage connections:

Three-Phase + Neutral: 400 VAC.

Single-Phase: 230 VAC.

Three-Phase supply hose with IP44 3PN+E 32A 400V connecting plug.

Differential magnetothermal, 4 poles, 25A, 300mA AC 6KA.

• N-REVT/1K. 1kW Three-Phase Variable Resistors Module.

Resistance: 3 x 150-500 Ohm.

Fuses: 3 x 2 A.

• N-CAR19T/3C. 3 x 300 VAr Three-Phase Configurable Inductances Module.

Configurable Star and Delta connection.

Three benches with three-phase capacitors of 2 µF each one.

Nominal voltage: 400 VAC.

Nominal power: 3 x (3 x 300) VAr.

• N-INDT/3C. 3 x 300 VAr Three-Phase Configurable Capacitors Module.

Configurable Star and Delta connection.

Three benches with three-phase inductances of 5 H each one.

Nominal voltage: 400 VAC.

Nominal power: 3 x (3x300) VAr.

• N-EALD. Network Analyzer Unit with Computer Data Acquisition.

ON-OFF switch.

Supply voltage: 230 VAC+N+GND.

Input terminals: Input connection with the measurement point.

Output terminals: Output connection with the measurement point.

RS-485 Communication port.

Fuses: 3x10 A.

Network Analyzer Display. It shows:

Voltages (phase-phase, phase-neutral, angles, frequency, unbalance, THD)

Currents (max. demand, unbalance, TDD, THD)

Active, reactive and apparent power

Power factor (inductive and capacitive load)

Maximum demand (import, export, voltages, currents, PF).

MAX/MIN values.

V and I individual harmonics and their spectrums.

Voltages and currents phasors diagrams.

Waveforms of voltages and currents and THD values.

State of digital signals, external pulse counters or installation point counters

Relay output status.

Load bar graph displays the amount, in percent with respect of the nominal defined by the user.

• N-PSM. Power Switch Module.

The voltage can be switch on and off manually or automatically.

Nominal voltage: 230 VAC+N+GND.

Frequency: 50/60Hz.

Control voltage: 24 VDC.

Nominal operating current: 15 A.

Functions: 2 pushbutton switches and remote control for switch-off relay.

Contacts:

Three normally open contacts.

Two auxiliary normally open contacts.

• N-REL09. Time Electronic Relay against Overcurrents (1.2 - 7 A).

Electronic thermal relay.

Overload protection.

Range 1.2 - 7 A.

Reset and test function.

1 NONC contact.

Temporization.

• All necessary cables to realize the practical exercises are included.

Cables and accessories, for normal operation.

Manuals:

This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

-Dimensions: 1380 x 840 x 600 mm approx. (54.33 x 33.07 x 23.62 inches approx.). -Weight: 80 Kg approx. (176 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Wiring star and delta connection with  capacitive and inductive loads.

2.- Overcurrent relay and power switch configuration and testing.

3.- Measurement of the active power consumed by a receiver in a pure resistive circuit.

4.- Measurement of the reactive power consumed by a receiver in a pure inductive circuit.

5.- Measurement of the reactive power consumed by a receiver in a pure capacitive circuit.

6.- Measurement of the apparent, active and reactive powers consumed by a receiver in a RL circuit.

7.- Measurement of the power factor of a  receiver in a RL circuit.

8.- Computation and wiring of the  capacitive load needed  to carry out the appropiate power factor correction.

9.- Comparison of the active energy consumed after the compensation.

10.- Comparison of the reactive energy consumed after the compensation.

11.- Measurement of power factor after the compensation.

12.- Determination of the first and second power maxima.

13.- Determination of the power maximum in the event of an asymmetric load.

Practical exercises with the recommended Control and Data Acquisition System Software for Electrical Machines, with SCADA, “EM-SCADA”:

14.- Real time current and voltage RMS values monitoring.

15.- Real time current and voltage waveforms measurement.

16.- Real time monitoring of the results.

17.- Obtaining the saved results.

- Several other exercises can be done and designed by the user.

Optional

AEL-MRPC/ICAI. Interactive Computer Aided Instruction Software System:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.