TENDER SPECIFICATIONS

Measurements:

Carbon monoxide CO (%).

Carbon dioxide CO2 (%).

HC gasoline, propano, methane (ppm).

Oxygen O2 (%).

Carbon monoxide CO adjusted (%).

Lambda calculation.

Oil temperature.

Technical Data:

Power Supply: 115/230 VAC - 50/60 HZ - 68 W.

Measure time: 12 seconds (maximal).

WarmUp time: 6 minutes (from 25 ºC).

Work Temperature: from +2 ºC to +40 ºC.

Humidity: 95 % max (non condensed).

Pressure: 800-1060 mbar.

CO (+/- 0.02 % abs +/-3 % rel): from 0 to 15%.

CO2 (+/- 0.3 % abs +/-3 % rel): from 0 to 20%.

HCg (+/- 4ppm abs +/-3 % rel): from 0 to 15000 ppm vol.

HCp (+/- 8ppm abs +/-3 % rel): from 0 to 30000 ppm vol.

O2 (+/- 0.1 % abs +/-5 % rel): from 0 to 25 %.

COc (+/- 1 % abs +/-3 % rel): from 0 to 20 %.

Lambda: from 0 to 9.999 rmp.

Oil temperature: 0 - 200 ºC.

Sample gas tube length: 6 m approx.

Sample gas tube diameter: 5 mm approx.

USB wire to computer (PC).

Control software.

Dimensions: 600 x 200 x 300 mm approx.  Weight: 5 Kg approx.