Tender Specifications

Bench-top unit with adjustable legs.

Anodized aluminum frame and panels made of painted steel.

The "MTP" unit mainly consists of:

Two stainless steel guides of 800 mm, which allow to displace the supports in the whole range. This allows the student to carry out the experiment of beams of different length.

Four test pieces with circular section, of 8 mm diameter, of different materials (steel, aluminum, brass, bronze). They have marks every 50 mm to make the measurement of the beam length easier.

Seven test pieces with different rectangular cross section made of stainless steel. They also have marks every 50 mm to make the measurement of the beam length easier.

Dial gauge of 0 – 10 mm to measure the deformations with 0.01 mm accuracy.

The elements required to perform the practical exercises are supplied:

Allen key of 3 mm to assemble the beams in the supports.

Set of weights adapted to the "MTP" unit with special hooks for each type of experiment:

Two weights of 10 N.

Four weights of 5 N.

Manuals: This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

Dimensions: 850 x 500 x 650 mm approx. (33.46 x 19.68 x 25.59 inches approx.)  Weight: 18 Kg approx. (39.68 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.-  Study of the beams' flexion for different sections and lengths.

2.-  Determination of the Elasticity Modulus for stainless steel.

3.-  Study of the relation between the torsional moment, beam length and torsion angle of one shaft.

4.-  Determination of the Rigidity Modulus for steel, bronze and aluminum.

Optional

MTP/ICAI. Interactive Computer Aided Instruction Software System:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.

BDAS. Basic Data Acquisition System and Sensors:

For being used with mechanical modules.

BDAS is designed to monitor the measurements of each mechanical module from a computer.