# TENDER SPECIFICATIONS

Base Service Units

FME00. Hydraulics Bench.

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Mobile hydraulic bench, made in fibreglass reinforced polyester, and mounted on wheels for its mobility.

Centrifugal pump, 0.37 KW, 30 – 80  l/min at 20.1-12.8 m, single phase 200 VAC – 240 VAC/50 Hz or 110 VAC – 127 VAC/60 Hz.

Runner made of stainless steel.

Sump tank capacity: 165  l.

Small channel: 8  l.

Flow measurement: volumetric tank, gauged from 0 to 7  l for low flow values and from 0 to 40  l for high flow values.

Control valve for regulating the flow.

Open channel to place the test module.

Measuring cylinder is provided for the measurement of small flow rates.

Remote hand-operating dump valve in the base of the volumetric tank.

Rapid and easy interchange of the different modules.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Recommended element:

- FME-DFM. Digital Flow Meter.

Dimensions: 1130 x 730 x 1000 mm approx. (44.48 x 28.74 x 39.36 inches approx.)   Weight: 70 kg approx. (154 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

- Flow measurement.

FME00/B. Basic Hydraulic Feed System.

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Centrifugal pump: 0.37 KW, 30 – 80  l/min at 20.1-12.8m., single-phase 200 VAC – 240 VAC/50 Hz or 110 VAC – 127 VAC/60 Hz.

Stainless steel impeller.

Tank capacity: 140  l approx.

Flowmeter.

Membrane type flow adjusting valve.

Safety switch ON/OFF.

Supports for accommodating the test module.

This unit incorporates wheels for its mobility.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Recommended element:

- FME-DFM. Digital Flow Meter.

Dimensions: 1000 x 600 x 700 mm approx.  (39.36 x 23.62 x 27.55 inches approx.)  Weight: 40 kg approx. (88 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

- Flow measurement.

Fluid Concepts

FME02. Flow over Weirs.

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Dimensions of the weirs: 230 x 4 x 160 mm.

Neckline angle in the V-shape weir: 90º.

Dimension of rectangular notch: 30 x 82 mm.

Scale of the level meter: 0 to 160 mm.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Dimensions: 400 x 160 x 600 mm approx. (15.74 x 6.29 x 23.62 inches approx.).   Weight: 7 kg approx. (15.43 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Study of the flow characteristics through a weir with a rectangular neckline, made on a thin wall.

2.- Study of the flow characteristics through a weir with a V-shape neckline, made on a thin wall.

FME04. Orifice Discharge.

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Transparent cylindrical tank.

Five type of mouthpieces: diaphragm, colloidal, 2 of Venturi and cylindrical.

Height of maximum load: 400 mm.

Easy and quick coupling system built-in.

Anodized aluminum structure.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Dimensions: 450 x 450 x 900 mm approx. (17.71 x 17.71 x 35.43 inches approx.).   Weight: 15 kg approx. (33 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

- Determination of the discharge coefficient for the mouthpiece of thin wall, Venturi type.

- Determination of the velocity coefficient for the mouthpiece of thin wall, Venturi type.

- Determination of the contraction coefficient for the mouthpiece of thin wall, Venturi type.

- Determination of the discharge coefficient for the mouthpiece of thin wall, diaphragm type.

- Determination of the velocity coefficient for the mouthpiece of thin wall, diaphragm type.

- Determination of the contraction coefficient for the mouthpiece of thin wall, diaphragm type.

- Determination of the discharge coefficient for the mouthpiece of thin wall, colloidal type.

- Determination of the velocity coefficient for the mouthpiece of thin wall, colloidal type.

- Determination of the contraction coefficient for the mouthpiece of thin wall, colloidal type.

- Determination of the discharge coefficient for the mouthpiece of thick wall, cylindrical type.

- Determination of the velocity coefficient for the mouthpiece of thick wall, cylindrical type.

- Determination of the contraction coefficient for the mouthpiece of thick wall, cylindrical type.

- Determination of the discharge coefficient for the mouthpiece of thick wall, Venturi type.

- Determination of the velocity coefficient for the mouthpiece of thick wall, Ventury type.

Determination of the contraction coefficient for the mouthpiece of thick wall, Ventury type.

Fluid Concepts

FME14. Free and Forced Vortex.

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Tank diameter: 300 mm.

Tank height: 300 mm

Mouthpieces orifice diameters: 8, 16 and 24 mm.

Distance between centers: 0, 30, 50, 70, 90 and 110 mm.

Pitot tube with measuring points at: 15, 20, 25 and 30 mm radius and a scale.

Measurement bridge.

Inlet pipes: 9 and 12.5 mm. diameter.

Diameter measurement system by Nonius.

Blind mouthpiece with X-shaped crosses.

Easy and quick coupling system built-in.

Anodized aluminum structure.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Dimensions: 600 x 550 x 1400 mm approx. (23.62 x 21.65 x 55.11 inches approx.).  Weight: 10 kg approx. (22 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Study of forced vortex without discharge orifice.

2.- Study of forced vortex with discharge orifice.

3.- Study of free vortex.

4.- Analysis of the influence of the jet inlet direction.

5.- Analysis of the influence of the vortex on the discharge velocity.

FME34. Fluid Statics and Manometry.

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

The module is mounted on an aluminum structure and painted Steel panels and consists of a vertical tank (made of methacrylate, diameter: 100 mm and height: 575 mm) containing water that is connected to different vertical manometer tubes (460 mm length):

“U” shape vertical tube.

Two parallel vertical tubes.

Vertical tube with variable section.

Vertical tube with a pivot that allows it to incline from 0 to 90º.

These tubes can be used individually or in combination for the different demonstrations.

Vernier hook and point gauge.

Piezometric tube.

Manual air pump.

Purge valve.

Plug to close the tank, so that it is not open to atmospheric pressure.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Dimensions: 700 x 350 x 800 mm approx. (27.55 x 13.77 x 31.49 inches approx.).  Weight: 15 kg approx. (33 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.-   To study the basic principles of hydrostatics and to demonstrate the behaviour of liquids at rest.

2.-   To use manometer tubes to measure differential pressure.

3.-   To use a manometer tube to measure head.

4.-   To use a ‘U’ tube manometer to measure pressure differences in a gas (air over liquid).

5.-   To use a U-shaped manometer for determining the differential pressure.

Fluid Concepts

6.-   To use liquids with different densities to change the ‘U’ tuve manometer sensitivity.

7.-   To use an inverted pressurized ‘U’ tube manometer to measure pressure differences in a liquid.

8.-   To use an inclined manometer with different inclinations.

9.-   Level measurement using Vernier hook and point gauge.

10.- To measure the liquid level using a scale.

11.- Demonstrating that the level of a free surface is not affected by the size or shape of the tube.

12.- Use of a piezometric tube to measure pressure.

13.- Observing the effect of a liquid in motion (losses due to friction).

FME35. Fluid Properties.

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Anodized aluminum structure and panels of painted steel.

Three hydrometers of resolution 0.002ºSG:

Hydrometer 0.8 ºSG - 1ºSG

Hydrometer 1 ºSG – 1.2 ºSG

Hydrometer 1.2 ºSG – 1.4 ºSG

Two hydrometer jars of 450 x 50 mm.

Aneroid barometer, range: 973 – 1047 mbar.

Thermometer with a range between -10 and 50 ºC.

Pycnometer of 50 ml.

Parallel plates capillary module.

Capillary tubes module with tubes of different size: 5 mm, 4 mm, 3 mm, 2.2 mm, 1.7 mm and 1.2 mm.

Two falling sphere viscometer tubes of 300 x 40 mm, with marks at 0, 25, 175, 200 and 220.

Set of stainless steel balls of different sizes: 3.175 mm, 2.381 mm and 1.588 mm.

Variable scale lever balance to be used with the Archimedes module, up to 310 gr.

Archimedes module: displacement vessel, bucket and cylinder.

Graduated cylinder made of glass (250 ml.)

Two beakers made of glass (600 ml.)

Digital chronometer.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Dimensions: 850 x 500 x 800 mm approx. (33.46 x 19.68 x 31.49 inches approx.).  Weight: 20 kg approx. (44 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.- To study the effect of capillary elevation between flat plates.

2.- To study and measure the effect of capillary elevation inside capillary tubes.

3.- To study and verify the Archimedes principle using a bucket and cylinder with a lever balance.

4.- To measure the fluid density and relative density of a liquid using a hydrometer and using a density bottle.

5.- To measure the atmospheric pressure using a barometer.

6.- To measure the fluid viscosity using a falling sphere viscometer.

7.- To measure the fluid temperature using an alcohol thermometer.

8.- Measuring of liquid levels.

FME36. Rotameter.

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Anodized aluminum structure and panel of painted steel.

Variable area rotameter with float.

Measurement range: 600 – 6000  l/h.

Fluid Concepts

Material: transparent PVC.

Accuracy class: 4.

Quick-plug for an easy connection.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Dimensions: 400 x 300 x 900 mm approx. (15.74 x 11.81 x 35.43 inches approx.).  Weight: 10 kg approx. (22 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Flow measurement.

General Fluid Applications

FME01. Impact of a Jet.

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Jet diameter: 8 mm.

Impact surfaces diameter: 40 mm.

Impact surfaces:

180° hemispherical surface.

120° curve surface.

90° flat surface.

A set of masses of 5, 10, 50 and 100 g is supplied.

Easy and quick coupling system built-in.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Dimensions: 250 x 250 x 500 mm approx. (9.84 x 9.84 x 19.68 inches approx.).   Weight: 5 kg approx. (11 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

- Impact against a flat surface.

- Impact against a curve surface of 120º.

- Impact against a hemispherical surface.

Use of the fast connectors.

FME08. Hydrostatic Pressure.

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Tank capacity: 5.5  l.

Distance between the suspended masses and the support point: 285 mm.

Area of the section: 0.007 m².

Total depth of the submerged quadrant: 160 mm.

Height of the support point on the quadrant: 100 mm.

A set of masses of different weights is supplied (4 of 100 gr, 1 of 50 gr, 5 of 10 gr, and 1 of 5 gr).

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Dimensions: 550 x 250 x 350 mm approx. (21.65 x 9.84 x 13.77 inches approx.).   Weight: 5 kg approx. (11 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.-  Determination of the center of pressures with an angle of 90°, partially submerged.

2.-  Determination of the resultant force with an angle of 90°, partially submerged.

3.-  Determination of the center of pressures, angle <> 90° partially submerged.

4.-  Determination of the equivalent force with an angle <>90° partially submerged.

5.-  Determination of the center of pressures with an angle of 90° totally submerged.

6.-  Determination of the resultant force with an angle of 90° totally submerged.

7.-  Determination of the center of pressures, angle <>90° totally submerged.

8.-  Determination of the resultant force, angle <>90° totally submerged.

9.-  Balance of momentum.

General Fluid Applications

FME10. Dead Weight Calibrator.

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Pressure manometer:

Bourdon type.

0 - 2.5 bar.

Masses (approximated weights):

0.5 kg.

1.0 kg.

2.5 kg.

5 kg.

Piston diameter: 18 mm.

Piston weight: 0.5 kg.

Anodized aluminum structure.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Dimensions: 500 x 400 x 500 mm approx. (19.68 x 15.74 x 19.68 inches approx.).   Weight: 10 kg approx. (22 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Bourdon type manometer calibration.

2.- Hysteresis curve determination.

FME11. Metacentric Height.

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Maximum angle: +/- 13°.

Corresponding lineal dimension: +/- 90 mm.

Dimension of the float:

Length: 353 mm.

Width: 204 mm.

Total height: 475 mm.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Dimensions: 750 x 400 x 750 mm approx. (29.52 x 15.74 x 29.52 inches approx.).   Weight: 5 kg approx. (11 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Study of the stability of a floating body. Angular displacements.

2.- Study of the stability of a floating body. Different positions of the center of gravity.

3.- Determination of the metacentric height.

General Fluid Applications

FME11-A. Metacentric Height of a "V" Shape Floating Body.

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Floating body made of methacrylate with V shape base.

Maximum angle: +/- 13º.

Corresponding linear dimension: +/- 90 mm.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Dimensions: 350 x 200 x 500 mm approx. (13.77 x 7.87 x 19.68 inches approx.).   Weight: 5 kg approx. (11 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Study of the stability of a floating body with V shaped base. Angular displacements.

2.- Study of the stability of a floating body with V shaped base. Different positions of the center of gravity.

3.- Determination of the metacentric height of a floating body with V shaped base.

FME11-B. Metacentric Height of a "U" Shape Floating Body.

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Floating body made of methacrylate with U shape base.

Maximum angle: +/- 13º.

Corresponding linear dimension: +/- 90 mm.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Dimensions: 350 x 200 x 500 mm approx. (13.77 x 7.87 x 19.68 inches approx.).   Weight: 5 kg approx. (11 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Study of the stability of a floating body with U shaped base. Angular displacements.

2.- Study of the stability of a floating body with U shaped base. Different positions of the center of gravity.

3.- Determination of the metacentric height of a floating body with U shaped base.

FME26. Depression Measurement System (vacuum gauge).

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Anodized aluminum structure.

Pressure-vacuum gauge adjusted from -1 to 0 bar.

Quick connections.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Dimensions: 220 x 110 x 420 mm approx. (8.66 x 4.33 x 16.53 inches approx.).  Weight: 2 kg approx. (4.4 pounds approx.).

General Fluid Applications

EXERCISES AND PRACTICAL POSSIBILITIES

1.- To measure the depression caused for the fluid aspiration by an hydraulic pump.

2.- We can observe the different negative readings due to the different methods of fluid aspiration for its subsequent impulsion.

FME32. Pitot Static Tube Module.

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Pitot static tube:

Head diameter: 2.5 mm.

Transparent pipe:

32 mm internal diameter and 430 mm length approx.

Hose connections.

Water manometer, 500 mm length.

Easy and quick coupling system built-in.

Anodized aluminum structure and panel of painted steel.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Dimensions: 800 x 450 x 700 mm approx. (31.49 x 17.71 x 27.55 inches approx.).  Weight: 15 kg approx. (33 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Study of the function of a pitot static tube.

2.- To use a pitot static tube.

3.- Determination of tube flow speed profiles.

4.- Demonstration that the flow speed is proportional to the pressure difference between the total pressure and the static pressure.

5.- Error determination in flow measurements using the Pitot tube as measurement instrument.

6.- Factor Cd determination in the Pitot tube.

Hydraulic Laws

FME03. Bernoulli's Theorem Demonstration.

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Manometer range: 0 to 300 mm of water.

Number of manometer tubes: 8.

Upstream diameter of the throat: 25 mm.

Narrowing:

Downstream: 21°.

Upstream: 10°.

Easy and quick coupling system built-in.

Anodized aluminum structure and panel of painted steel.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Dimensions: 800 x 450 x 700 mm approx. (31.49 x 17.71 x 27.55 inches approx.).   Weight: 15 kg approx. (33 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

- Determination of the exact section in Venturi’s tube.

- Demonstration of Bernoulli’s Theorem. Divergent-convergent position.

- Determination of Bernoulli’s Theorem equation. Convergent-divergent position.

- Observation of differences between convergent and divergent position.

FME22. Venturi, Bernoulli and Cavitation Unit.

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Manometer (Bourdon type), range: 0 – 2.5 bar.

Manometer (Bourdon type), range: 0 – -1 bar.

Two tanks, height: 135 mm and internal diameter: 64 mm.

Venturi tube with 6 tappings (Divergent/Convergent).

Differential manometers: 0 – 500 mm.

Five manometric tubes.

Easy and quick coupling system built-in.

Anodized aluminum structure and panels of painted steel.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Dimensions: 750 x 400 x 850 mm approx. (29.52 x 15.74 x 33.46 inches approx.).  Weight: 10 kg approx. (22 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.- How to fill the manometric tubes.

2.- Flow calculation.

3.- Determination of the exact section in Venturi’s tube. Bernoulli’s theorem study.

4.- Cavitation study.

5.- Pressure reduction in a tank.

6.- Aspiration pump.

7.- Aspiration pump for mixing two liquids.

8.- Using for air and water mixing.

Hydraulic Laws

FME06. Osborne - Reynolds' Demonstration.

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Tube inner diameter: 10 mm.

Tube outer diameter: 13 mm.

Visualization pipe length: 700 mm.

Capacity of the dye tank: 0.3  l.

Tank capacity: 10  l.

Flow control valve: diaphragm type.

The coloured fluid is regulated with a needle valve.

Easy and quick coupling system built-in.

Anodized aluminum structure and panels of painted steel.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Recommended element:

- WHM. Water Heater Module.

Dimensions:  450 x 450 x 1250 mm approx. (17.71 x 17.71 x 49.21 inches approx.).   Weight: 20 kg approx. (44 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

- Observation of the laminar, transition and turbulent regime.

- Study of the velocity profile, reproducing the Osborne-Reynolds’s experiment.

- Reynolds’s number calculation.

FME31. Horizontal Osborne-Reynolds Demonstration.

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

This module is mounted on an anodized aluminum structure with painted steel panel.

Methacrylate test pipe with an airfoil-shaped inlet section:

Inner diameter: 16 mm.

External diameter: 20 mm.

Length: 750 mm.

Water supply tank with level fitting and connection for its feeding. It has a section that makes it possible to generate a constant pressure at the tank inlet. Capacity: 2.4  l.

Dye or vegetable colouring tank with a valve and an injection needle, tank capacity: 0.4  l.

Colouring matter injection is regulated with a needle valve.

Control valve to adjust the water flow in the experiments.

Easy and quick coupling system built-in.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Dimensions: 1100 x 400 x 700 mm approx. (43.30 x 15.74 x 27.55 inches approx.).  Weight: 20 kg approx. (44 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Observation of laminar, transition and turbulent flows.

2.- Association of laminar, transition and turbulent flows with their corresponding Reynolds number.

3.- Observation of the parabolic velocity profile.

Hydraulic Laws

FME24. Unit for the Study of Porous Beds in Venturi Tubes (Darcy´s Equation).

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Manometer range: 0-300 mm. of water.

Number of manometric tubes: 8.

Strangulation diameter upstream: 25 mm.

Narrowing:

Upstream: 10º.

Downstream: 21º.

Venturi’s tube with Pitot tube.

Venturi’s tube with porous bed of a grain diameter of 1.0 to 1.5 mm (FME24/A).

Venturi’s tube with porous bed of a grain diameter of 2.5 to 3.5 mm (FME24/B).

Venturi’s tube with porous bed of a grain diameter of 5.5 to 7.0 mm (FME24/C).

Easy and quick coupling system built-in.

Anodized aluminum structure and panels of painted steel.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Dimensions: 800 x 450 x 700 mm approx. (31.49 x 17.71 x 27.55 inches approx.).  Weight: 15 kg approx. (33 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Demonstration of Bernoulli’s theorem and its limitations in divergent - convergent position.

2.- Demonstration of Bernoulli’s theorem and its limitations in convergent-divergent position.

3.- Direct measurement of the static height and of the total distribution of heights in Venturi’s tubes.

4.- Determination of the exact section in a Venturi’s tube.

5.- Head losses in the porous bed (elements FME24/A, FME24/B and FME24/C).

FME33. Pascal´s Module.

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Anodized aluminum profile frame and painted steel panel that guarantees good stability and resistance to the environment.

Three vessels of 230 mm high:

Straight shaped vessel with internal diameter of 26 mm.

Conical vessel with internal diameter from 26 mm to 80 mm.

Inverted conical vessel with internal diameter from 26 mm to 10 mm.

Support for the vessel and membrane.

Lever arm and spirit level to measure the weight at the base of the vessel.

Fastening nut for levelling.

Masses set.

Vertical rod with portable indicator to fix the fluid level in the vessels.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Dimensions: 550 x 350 x 500 mm approx. (21.65 x 13.77 x 19.68 inches approx.).  Weight: 7 kg approx. (15 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Demonstration of Pascal's principle by comparing three vessels of different shape.

2.- Determining the hydrostatic pressure.

3.- Determining the linear relation between pressure and filling head of the vessel.

Hydraulic Demonstration

FME09. Flow Visualization in Channels.

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Capacity of the dye tank: 0.3  l.

Width/length of the channel approx.: 15/630 mm.

Depth of channel approx.: 150 mm.

Damping tank that eliminates the turbulences.

Hydrodynamic models:

Two lengthened.

Two circular of 25 and 50 mm diameter.

Rectangle with rounded edges.

Wedge.

Easy and quick coupling system built-in.

Anodized aluminum structure.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Dimensions: 900 x 450 x 500 mm approx. (35.43 x 17.71 x 19.68 inches approx.).   Weight: 7 kg approx. (15 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.-  Leakage of liquids by thin-wall weirs.

2.-  Liquid leakage by thick-wall weirs.

3.-  Models with wing profile submerged in a fluid current.

4.-  Circular models submerged in a fluid current.

5.-  Demonstration of the phenomenon associated to the flow in open channels.

6.-  Visualization of the flow lines around different submerged hydrodynamic models.

FME20. Laminar Flow Demonstration.

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Capacity of dye tank: 0.3  l.

Width/length of the table: 400/210 mm.

Depth of the table: adjustable depending on the models.

Hydrodynamic models:

Two circular ones of 25 and 50 mm. diameter.

Two rectangular ones of 25 x 25 and 50 x 50 mm.

Wedge.

Easy and quick coupling system built-in.

Anodized aluminum structure.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Dimensions: 750 x 400 x 650 mm approx. (34.25 x 17.71 x 15.74 inches approx.).  Weight: 10 kg approx. (22 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Ideal flow around a submerged cylinder.

2.- Ideal flow around a submerged profile.

3.- Ideal flow around a body in peak.

4.- Ideal flow in a convergent channel.

5.- Ideal flow in a divergent channel.

6.- Ideal flow in an elbow of 90º.

7.- Ideal flow in a sudden contraction.

8.- Ideal flow in a sudden broadening.

9.- Substitution of a line of current for a solid edge.

Hydraulic Demonstration

FME30. Transparent Vortex Flow Meter.

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Frame made of anodized aluminum and panels of painted steel.

PVC pipe to connect to the Hydraulics Bench (FME00) or the Basic Hydraulic Feed System (FME00/B).

Transparent vortex flow meter with voltage measurement.

Variable-area flow meter (rotameter), range: 150 – 1600  l/h.

Control valve to regulate the flow.

Digital precision balance:

Range: 0 – 2000 gr.

Precision: 1 gr.

Graduated glass vessel with a capacity of 2  l.

Chronometer.

Quick connection system.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Dimensions: 900 x 570 x 900 mm approx. (35.43 x 22.44 x 35.43 inches approx.).  Weight: 30 kg approx. (66.13 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Study and experiments with a transparent vortex flow meter.

2.- Study and experiments with a variable area flow meter.

3.- Measurement of volumetric volume flow rate.

4.- Measurement of gravimetric volume flow rate.

5.- Comparison of methods on several volumetric and mass flow measurements.

6.- Flow meters calibration.

7.- Comparison among different flowmeters.

FME30/I. Vortex Flow Meter.

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Frame made of anodized aluminum and panels of painted steel.

PVC pipe to connect to the Hydraulics Bench (FME00) or the Basic Hydraulic Feed System (FME00/B).

Vortex type flow meter:

Range: 4.5  l/min – 50  l/min.

Precision: ±2 % full scale.

Repeatability: ±0.25 % full scale.

Alarm warning possibility when the flow is below a reference point.

Bright LED display of 7.62 mm (0.3”).

Variable-area flow meter (rotameter), range: 150 – 1600  l/h.

Control valve to regulate the flow.

Digital precision balance:

Range: 0 – 2000 gr.

Precision: 1 gr.

Graduated glass vessel with a capacity of 2  l.

Chronometer.

Quick connection system.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Dimensions: 900 x 570 x 900 mm approx. (35.43 x 22.44 x 35.43 inches approx.).  Weight: 30 kg approx. (66.13 pounds approx.).

Hydraulic Demonstration

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Study and experiments with an industrial vortex flow meter.

2.- Study and experiments with a variable area flow meter.

3.- Measurement of volumetric volume flow rate.

4.- Measurement of gravimetric volume flow rate.

5.- Comparison of methods on several volumetric and mass flow measurements.

6.- Flow meters calibration.

7.- Comparison among different flowmeters.

FME15. Water Hammer.

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Constant level deposit, in methacrylate.

Unload deposit, in methacrylate.

Pipe circuits in PVC.

Valves to select the circuit.

Two adjustable equilibrium chimneys and subjection clips.

Connections system to the Hydraulics Bench (FME00) or Basic Hydraulic Feed System (FME00/B) with fast plugs.

Easy and quick coupling system built-in.

Anodized aluminum structure.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Dimensions: 1215 x 270 x 1430 mm approx. (47.83 x 10.62 x 56.29 inches approx.).  Weight: 15 kg approx. (33 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Subduing of the water hammer effects.

2.- Study of the subduing depending on the diameter of the chimney.

3.- Calculations of the energy losses in pipes.

FME19. Cavitation Phenomenon Demonstration.

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Manometer range: 0 to 2.5 bar.

Vacuum gauge range: from-1 to 0 bar.

Throat section: 36 mm².

Normal section: 150 mm².

Easy and quick coupling system built-in.

Anodized aluminum structure and panel of painted steel.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Dimensions: 750 x 400 x 650 mm approx. (29.52 x 15.74 x 25.59 inches approx.).  Weight: 5 kg approx. (11 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Study of cavitation.

2.- Visualization of the cavitation phenomenon with forced  conduction.

Hydraulic Demonstration

FME18. Flow Meter Demonstration.

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Manometer range: 0 to 500 mm of water column.

Number of manometric tubes: 8.

Orifice plate diameter: 25 mm.

Flowmeter: 2 to 30 l/min.

Venturi dimensions:

Throat diameter: 20 mm.

Upstream pipe diameter: 32 mm.

Downstream taper: 21°.

Upstream taper: 14°.

Orifice Plate dimensions:

Upstream pipe diameter: 35 mm.

Downstream orifice diameter: 19 mm.

Easy and quick coupling system built-in.

Anodized aluminum structure and panel of painted steel.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Dimensions: 750 x 450 x 950 mm approx. (29.52 x 17.71 x 37.40 inches approx.).  Weight: 10 kg approx. (22 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Filling of the manometric tubes.

2.- Determination of the error in flow measurements using the Venturi.

3.- Determination of the Cd factor in the Venturi.

4.- Determination of the strangulation in the Venturi.

5.- Determination of the error in flow measurements using the orifice plate.

6.- Determination of the Cd factor in the orifice plate.

7.- Determination of the effective area in an orifice plate.

8.- Comparison of the energy loss in the three different elements.

9.- Comparison among the Venturi, the orifice plate and the flowmeter.

FME25. Flow channel, 1 m length.

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Channel of rectangular section with transparent walls in methacrylate, length: 1 m.

Rigid and flexible pipes.

Regulating valves.

Storage tank.

Tank with soothing of flow.

Easy and quick coupling system built-in.

Anodized aluminum structure.

Wide range of available accessories.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Dimensions: 1500 x 500 x 500 mm. approx. (59.04 x 19.68 x 19.68 inches approx.).  Weight: 40 Kg. approx. (88 pounds approx.).

Hydraulic Demonstration

Available Accessories for the Flow Channel, 1 m length (FME25):

FME25TP. Pitot tube for FME25:

Pitot tube with a panel (with two manometric tubes) that is introduced in the channel to measure pressures and obtain the speed and flow rates at different points of the channel.

The FME25TP accessory consists of a Pitot tube mounted on a movable XYZ stand, which can move all the length and breadth of the flow channel, and a panel with two manometric tubes to measure static and total pressure. The difference between both pressures allows us to calculate the speed of the fluid and, knowing the section, the flow can be calculated at any point.

Practical possibilities:

1.- Measurement of the flow rate with a Pitot tube.

2.- Determination of the static and total pressure.

3.- Filling of manometric tubes.

FME25CV. Vertical flat gate for FME25:

The FME25CV accessory is a vertical flat gate made of PVC that is located at the outlet of the channel to avoid the flow of fluid.

FME25SDL. Syphon spillway for FME25:

One way to regulate the flow in a channel is by using a syphon. When the level exceeds a specific height, water flows through the syphon and the level is regulated upstream of the syphon.

The FME25SDL accessory can be fixed to any part of the channel.

Practical possibilities:

1.- Understanding the operation of a syphon with free discharge.

2.- Calculation of the maximum flow admitted by the syphon.

3.- Level control through a syphon with free discharge.

FME25SDS. Self-regulating syphon for FME25:

One way to regulate the flow in a channel is by using a syphon. When the level exceeds a specific height, water flows through the syphon and the level is regulated upstream of the syphon.

The FME25SDS accessory can be fixed to any part of the channel.

Practical possibilities:

1.- Understanding the operation of a syphon with submerged discharge.

2.- Calculation of the maximum flow admitted by the syphon.

3.- Level control through a syphon with submerged discharge.

FME25RM. Scale to measure the water level (limnimeter) for FME25:

The limnimeter is used to measure the water level in the flow channel.

The instrument consists of several feeler tips that can move along a graduated scale from 0 to 500 mm. to obtain the level.

The scale is divided into tenth parts of a millimeter (adjustable Vernier scale).

Main metallic elements are made of stainless steel and this device can move along the whole channel.

Practical possibilities:

1.- Use of a limnimeter.

2.- Measuring the water level in the flow channel.

FME25PR. Adjustable undershot weir for FME25:

One way to regulate the flow in a channel is by using control gates. When the gate is totally closed, no water flows, and when the gate is open, water starts to flow through the channel.

The FME25PR accessory consists of a PVC gate mounted on a frame that can be displaced along the channel.

This system allows the gate to be fixed to the desired height and to measure that height. It has flexible lateral reinforcements that guarantee watertightness.

Practical possibilities:

1.- Flow control with gates.

2.- Observation of discharge processes when using a weir.

3.- Observation of alternating changes during the discharge.

Hydraulic Demonstration

FME25VD. Sharp crested discharge weirs (two different models) for FME25:

Sharp crested weirs are hydraulic weirs, generally used to measure flow rates. Their name is due to the fact that the discharge is done through a plate whose profile, regardless of its shape, ends in a sharp edge.

The FME25VD accessory includes 2 weirs (a V shaped and a U shaped one) made of PVC lodged in slots, reinforced with flexible rubber, designed for that purpose at the outlet of the channel, guaranteeing watertightness.

Practical possibilities:

1.- Comparison between the main types of weirs.

2.- Measurement of the flow rate with a triangular sharp-crested weir (V shaped).

3.- Measurement of the flow rate with a rectangular sharp-crested weir (U shaped).

4.- Comparison between the theoretical and experimental flows.

FME25VG. Broad-crested weirs (two different models) for FME25:

Broad crested weirs have a lower discharge capacity for the same volume of water than sharp crested weirs. They are most frequently used as level control structures, although they can also be calibrated and used as flow measurement structures.

The FME25VG accessory includes two broad crested weirs, made of PVC, which can be fixed to any part of the bottom of the channel. The edge of one of the weirs is rounded and the edge of the other one is straight. Both weirs have flexible lateral reinforcements that guarantee watertightness.

Practical possibilities:

1.- Measurement of the flow rate with a broad crested weir.

2.- Comparison between the theoretical and experimental flows.

FME25PV. Ogee-created weir for FME25:

The Ogee weir is a fixed weir, that is to say, it does not allow regulation of the water surface profile.

They are used to divert flow rates since, if it is compared to other types of weirs, the special shape of their crest enables the maximum discharge for the same water level.

It is made of PVC, can be fixed to any part of the bottom of the channel and has flexible lateral reinforcements that guarantee watertightness.

Practical possibilities:

1.- Measurement of the flow rate with an Ogee-crested weir.

2.- Comparison between the theoretical and experimental flows.

FME25CC. Closed Channel Study for FME25:

The flow of water in a duct can be open channel flow or pipe  flow, which is known as a closed channel.

The FME25CC accessory includes an upper limit of PMMA, so that a high enough water level is achieved for the flow to pass through the whole study section and thus be able to determine the pressures in a pipe.

It has flexible lateral reinforcements that guarantee watertightness.

Practical possibilities:

1.- Study of the flow in closed channels.

2.- Pressure measurement.

3.- Determination of the flow rate. It has flexible lateral reinforcements that guarantee watertightness.

EXERCISES AND PRACTICAL POSSIBILITIES

1.-   Study of the fundamental aspects of fluid flow.

Practical possibilities depending on the accessories used:

2.-   Measurement of water level and velocity along the channel.

3.-   Measurement of flow rate using a Pitot tube.

4.-   Determination of the static and total pressure.

5.-   Use of hydraulic structures to control level.

6.-   Study of the effects of gradual and sudden changes in cross section (energy losses).

7.-   Use of a contraction to measure flow.

8.-   Use of hydraulic structures to measure flow in an open channel.

9.-   Study of flow patterns associated with flow around hydraulic structures.

10.- Comparison between the theoretical and experimental flow.

FME17. Orifice and Free Jet Flow.

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Orifices with diameters of 3.5 and 6 mm.

Jet trajectory Probes: 8.

Maximum height: 500 mm.

Easy and quick coupling system built-in.

Anodized aluminum structure.

Hydraulic Demonstration

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Dimensions: 600 x 550 x 1400 mm approx. (23.62 x 21.65 x 55.11 inches approx.).  Weight: 10 kg approx. (22 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Determination of the orifice velocity coefficient.

2.- Obtaining of the orifice discharge coefficient in permanent regime.

3.- Obtaining of the orifice discharge coefficient in variable regime.

4.- Obtaining of the tank discharge time.

Pipes

FME05. Energy Losses in Bends.

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Range of the two Bourdon type manometers: 0 to 2.5 bar.

Differential manometers range: 0 to 500 mm.

Number of manometric tubes: 12.

PVC Rigid pipes:

Internal diameter: 25 mm.

External diameter: 32 mm.

Flexible pipes:

Pressure taking-differential manometer. External diameter: 10 mm.

Pressurizing equipment. External diameter: 6 mm.

Drain. External diameter: 25 mm.

Fittings:

Miter (90º angle).

90º curve.

90º medium elbow.

90º short elbow.

90º long elbow.

Broadening of 25/40.

Narrowing of 40/25.

Valves:

Membrane valves. Diameter: 25 mm.

Antireturn: 6 mm.

Easy and quick coupling system built-in.

Anodized aluminum structure and panel of painted steel.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Dimensions: 750 x 550 x 950 mm approx. (29.52 x 21.65 x 37.40 inches approx.).  Weight: 10 kg approx. (22 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

- Filling of the manometric tubes.

- Measurement of the flow.

- Measurement of load losses for a short elbow of 90º.

- Measurement of load losses for a medium elbow of 90º.

- Measurement of load losses for a curve of 90º.

- Measurement of load losses for a broadening of 25/40.

- Measurement of load losses for a narrowing 40/25.

- Measurement of load losses for a miter type abrupt direction change.

- Measurement of load losses for a membrane valve.

FME07. Energy Losses in Pipes.

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Test pipe of 4 mm of inner diameter, 6 mm of external diameter and 500 mm of length.

Differential manometer of water column.

Pipes

Manometer scale: 0 to 500 mm (water).

Two Bourdon type manometers, range: 0 to 2 bar.

Constant height tank.

Easy and quick coupling system built-in.

Anodized aluminum structure and panels of painted steel.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Dimensions: 330 x 330 x 900 mm approx. (12.99 x 12.99 x 35.43 inches approx.)  Weight: 30 kg approx. (66 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

- Energy loss in pipes for a turbulent regime.

- Determination of the energy loss in a turbulent regime.

- Determination of the number of Reynolds for a turbulent regime.

- Energy loss in pipes for a laminar regime.

- Determination of the energy loss factor f for a pipe in laminar regime.

- Determination of Reynolds number for the laminar regime.

- Determination of the kinematic viscosity of water.

FME23. Basic Pipe Network Unit.

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Anodized aluminum structure where the pipe network is located and the subjection panel of the manometers.

Test pipes:

Three PVC pipes, with different diameters.

One methacrylate pipe.

Eight pressure intakes, connected to a manometric tubes panel of pressurized water.

Pressurization system.

Manometric tubes panel:

Number of manometric tubes: 8.

Range: 0 to 470 mm of water.

Inlet pipe. Outlet pipe.

Regulation valves for controlling the flow through the network.

Adjustable legs for leveling the unit.

Easy and quick coupling system built-in.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Dimensions: 600 x 350 x 800 mm approx. (23.62 x 13.77 x 31.49 inches approx.).  Weight: 30 kg approx. (66 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Load loss in a PVC pipe.

2.- Load loss in a methacrylate pipe.

3.- Study of the load loss in pipes made of the same material.

4.- Study of the load loss depending on the material.

5.- Friction coefficient in a PVC pipe.

6.- Friction coefficient in a methacrylate pipe.

7.- Study of the friction coefficient depending on the material.

8.- Study of the friction coefficient depending on the diameter.

9.- Configuration of network in parallel for pipes of the same material but different diameter.

10.- Configuration of network in parallel for pipes of the same diameter but different material.

Pipes

AFT/P. Fluid Friction in Pipes Unit.

The "AFT/P" unit mainly consists of:

Anodized aluminum frame and panels made of painted steel.

The unit includes wheels to facilitate its mobility.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

This unit allows the detailed study of fluid friction pressure losses, which occur when a non-compressible fluid flows through pipes, valves, pipe fittings and flow metering devices.

Five pipes of different diameter, material and roughness:

Rough pipe (PVC): external diameter: 25 mm and internal diameter: 17 mm.

Rough pipe (PVC): external diameter: 32 mm and internal diameter: 23 mm.

Smooth pipe (PMMA): external diameter: 10 mm and internal diameter: 6.5 mm.

Smooth pipe (PVC): external diameter: 20 mm and internal diameter: 16.5 mm.

Smooth pipe (PVC): external diameter: 32 mm and internal diameter: 26.5 mm.

Types of valves:

Angle-seat valve: internal diameter: 20 mm.

Gate valve: internal diameter: 20 mm.

Diaphragm valve: internal diameter: 20 mm.

Ball valve: internal diameter: 20 mm.

Types of couplings:

In-line strainer, internal diameter: 20 mm.

Sudden widening. Its section changes from 25 mm to 40 mm.

Sudden contraction. Its section changes from 40 mm to 25 mm.

90º elbow: inner diameter: 20 mm.

“T” junction: inner diameter: 20 mm.

45º elbow: inner diameter: 20 mm.

45º “T” junction: inner diameter: 20 mm.

Symmetrical “Y” branch: inner diameter of each pipe: 20 mm.

Double 90º elbow: inner diameter: 20 mm.

Special couplings (PMMA):

Pitot tube: length:30 mm, external diameter: 4 mm and internal diameter: 2.5 mm.

Venturi tube: length: 180 mm, larger section: 32 mm and smaller section: 20 mm.

Diaphragm with measuring plate: larger diameter: 25 mm and smaller diameter: 20 mm.

The unit includes several ball valves to conduct the water flow through a certain pipe of the circuit and a regulation valve to regulate the flow that runs through such pipe.

Thirty four pressure tappings with quick action connections.

Two water manometers, range: 0 – 1000 mm.

Two Bourdon manometers, range: 0 – 2.5 bar.

- Flowmeter 600 – 6000  l/h, "C-F6000".

Manuals: This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

Dimensions: 2100 x 850 x 1000 mm approx. (82.67 x 33.46 x 39.37 inches approx). Weight: 150 kg approx. (330.7 pounds approx).

EXERCISES AND PRACTICAL POSSIBILITIES

1.-  Determination of pressure loss due to friction in a rough pipe with an internal diameter of 17 mm.

2.-  Determination of pressure loss due to friction in a rough pipe with an internal diameter of 23 mm.

3.-  Determination of pressure loss due to friction in a smooth pipe with an internal diameter of 6.5 mm.

4.-  Determination of pressure loss due to friction in a smooth pipe with an internal diameter of 16.5 mm.

5.-  Determination of pressure loss due to friction in a smooth pipe with an internal diameter of 26.5 mm.

6.-  Study of the influence of the diameter in the pressure loss due to friction in rough pipes.

7.-  Study of the influence of the diameter in the pressure loss due to friction in smooth pipes.

8.-  Study of the influence of the roughness in the pressure loss.

9.-  Determination of the friction coefficient in a rough pipe with an internal diameter of 17 mm.

10.-Determination of the friction coefficient in a rough pipe with an internal diameter of 23 mm.

11.-Determination of the friction coefficient in a smooth pipe with an internal diameter of 6.5 mm.

12.-Determination of the friction coefficient in a smooth pipe with an internal diameter of 16.5 mm.

13.-Determination of the friction coefficient in a smooth pipe with an internal diameter of 26.5 mm.

14.-Study of the influence of the diameter in the friction coefficient in rough pipes.

15.-Study of the influence of the diameter in the friction coefficient in smooth pipes.

16.-Comparison of the friction coefficient in smooth and rough pipes.

17.-Determination of pressure loss in an angle-seat valve.

Pipes

18.-Determination of pressure loss in a gate valve.

19.-Determination of pressure loss in a diaphragm valve.

20.-Determination of pressure loss in a ball valve.

21.-Comparison of pressure loss in different types of valves.

22.-Determination of pressure loss in an in-line strainer.

23.-Determination of pressure loss in a 90º elbow.

24.-Determination of pressure loss in a double 90º elbow.

25.-Determination of pressure loss in a 45º elbow.

26.-Determination of pressure loss in a 45º “T”.

27.-Determination of pressure loss in a symmetrical “Y” branch.

28.-Determination of pressure loss in a narrowing.

29.-Determination of pressure loss in a gradual widening.

30.-Determination of pressure loss in a diaphragm.

31.-Comparison of pressure loss in the different fittings.

32.-Measurement of the flow with the Venturi tube.

33.-Determination of the discharge coefficient, Cd, in the Venturi tube.

34.-Measurement of the flow with the Pitot tube.

35.-Determination of the discharge coefficient, Cd, in the Pitot tube.

36.-Comparison between the flow measured in the Venturi and Pitot tubes.

Additional practical possibilities:

37.-Study of the relationship between pressure losses due to fluid friction and the water flow rate.

38.-Determining the relationship between the pipe friction coefficients and Reynolds number for flow through a pipe with roughened bore.

39.-Determining of the resistance coefficients for bends, enlargements and contractions.

40.-Determining of characteristic curves of valves and fittings.

Hydraulic machines: Pumps

FME12. Series/Parallel Pumps.

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Centrifugal pump: 0.37 KW, 30 – 80 l/min at 20.1 – 12.8 m, single-phase 200 VAC – 240 VAC/50 Hz or 110 VAC – 127 VAC/60 Hz.

Absolute pressure manometer placed at the pump input, range - 1 to 3 bar.

Two manometers (manometric pressure), one of them placed in the discharge and the other one in the discharge accessory, range: 0 – 4 bar.

Membrane valve for flow regulating.

Two way valve: 2 positions: open or close.

Accessories:

Two flexible pipes with quick connections.

Reinforced pipe with quick connections.

Discharge accessory.

Easy and quick coupling system built-in.

Anodized aluminum structure and panels of painted steel.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Dimensions: FME12 module: 500 x 400 x 400 mm approx. (19.68 x 15.74 x 15.74 inches approx.).

Discharge accessory: 500 x 400 x 250 mm approx. (19.68 x 15.74 x 9.84 inches approx.).

Weight: 20 kg approx. (44 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.-  Water flow calculation.

2.-  H (Q) curve obtaining of a centrifugal pump.

3.-  Series coupling of two pumps with the same characteristics.

4.-  Parallel coupling of two pumps with the same characteristics.

FME13. Centrifugal Pump Characteristics.

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Centrifugal pump: 0.37 KW, 30 – 80  l/min at 20.1– 12.8 m with speed variator.

Bourdon type manometers.

Control panel for the variator, allowing to modify the speed, with visualization display that allows to know the rpm and the power consumed, and with on/off switch.

Discharge accessory, with manometer, flow control valve and diffuser.

Vacuum meter.

Easy and quick coupling system built-in.

Anodized aluminum structure and panels of painted steel.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Dimensions: 450 x 500 x 1250 mm approx. (17.71 x 19.68 x 49.21 inches approx.).  Weight: 40 kg approx. (88 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.-  Obtaining of the curves H (Q), N (Q), Eff% (Q) of a centrifugal pump.

2.-  Making of the map of a centrifugal pump.

3.-  Representation of the adimensional curves H*, N* and rpm*.

4.-  Series coupling of two pumps of similar characteristics.

5.-  Series coupling of two pumps of different characteristics.

6.-  Parallel coupling of two pumps of similar characteristics.

7.-  Parallel coupling of two pumps of different characteristics.

Hydraulic machines: Turbines

FME27. Axial Flow Turbine.

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Nozzle:

Inlet diameter of the throat: 2.5 mm.

Outlet diameter of the throat: 2.5 mm.

Discharge angle: 20º and 30º.

Turbine rotor:

External diameter: 53 mm.

Internal diameter: 45 mm.

Number of blades: 40.

Inlet angle of the blades: 40º.

Outlet angle of the blades: 40º.

Used material: Brass.

Brake:

Pulley diameter: 60 mm.

Real diameter: 50 mm.

Bourdon type manometer.

Eight ball valves.

Easy and quick coupling system built-in.

Anodized aluminum structure.

Tachometer.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Dimensions: 550 x 300 x 600 mm approx. (21.65 x 11.81 x 23.62 inches approx.).  Weight: 20 kg approx. (44 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Flow calculation.

2.- Determination of the discharge coefficient of the nozzle.

3.- Determination of the curve N (Q,n), Pm (Q,n) and  (Q,n); (20º nozzle).

4.- Determination of the curve N (Q,n), Pm (Q,n) and  (Q,n); (30º nozzle).

5.- Adimensional analysis.

FME16. Pelton Turbine.

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Speed range: 0 - 2000 rpm.

Power: 10 W.

Manometer range: 0 – 2.5 bar.

Number of buckets: 16.

Drum radius: 30 mm.

Dynamometers range: 0 – 20 N.

Easy and quick coupling system built-in.

Anodized aluminum structure.

Tachometer.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Dimensions: 750 x 400 x 750 mm approx. (29.52 x 15.74 x 29.52 inches approx.).  Weight: 15 kg approx. (33 pounds approx.).

Hydraulic machines: Turbines

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Determination of the operative characteristics of the Pelton Turbine.

2.- Determination of the operation mechanical curves.

3.- Determination of the operation hydraulic curves.

4.- Adimensionalization.

FME28. Francis Turbine.

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Functional model of Francis turbine.

Velocity range: 0 – 1000 rpm. Power: 5 W.

Diameter of the turbine: 52 mm.

Number of blades on the turbine: 15.

Number of adjustable guide vanes of the distributor: 10.

Manometer range: 0-250 mbar.

Braking system connected to 2 dynamometers:

Dynamometers range: 0 – 10 N.

Feed chamber.

Easy and quick coupling system built-in.

Anodized aluminum structure.

Tachometer.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Dimensions: 500 x 350 x 600 mm approx. (19.68 x 13.77 x 23.62 inches approx.).  Weight: 20 kg approx. (44 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.- To determine the operating characteristics of a Francis turbine at different velocities.

2.- Determination of the typical turbine curves (operating mechanical curves and operating hydraulic curves).

3.- Turbine power output versus speed and flow rate at various heads.

4.- Effect of guide vane setting on turbine performance.

5.- Adimensionalization.

FME29. Kaplan Turbine.

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Functional model of Kaplan Turbine.

Velocity range: 0 – 1000 rpm. Power: 10 W.

Number of blades of the turbine: 4.

Turbine diameter: 52 mm.

Number of adjustable guide vanes of the distributor: 8.

Manometer range: 0 – 200 mm. of water.

Braking system connected to 2 dynamometers:

Dynamometers range: 0 – 10 N.

Feed chamber.

Easy and quick coupling system built-in.

Anodized aluminum structure.

Tachometer.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Hydraulic machines: Turbines

Dimensions: 500 x 350 x 600 mm approx. (19.68 x 13.77 x 23.62 inches approx.).  Weight: 20 kg approx. (44 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Determination of the operative characteristics of Kaplan Turbine at different velocities.

2.- Flow calculation.

3.- Determination of the operation mechanical curves.

4.- Determination of the operation hydraulic curves.

5.- Adimensional analysis.

FME21. Radial Flow Turbine.

Structural Specifications:

Stainless frame.

Screws, nuts, plates and all the metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Quick connections for adaptation to feed hydraulics source.

Technical Data:

Nozzles:

Inlet diameter: 21 mm.

Outlet diameter: 2.0 mm.

Discharge angle: 180º.

Turbine rotor:

External diameter: 69 mm.

Internal diameter: 40 mm.

Number of nozzles: 2.

Inlet angle to the nozzle: 180º.

Outlet angle to the nozzle: 180º.

Used material: aluminum.

Brake:

Pulley diameter: 60 mm.

Effective diameter: 50 mm.

Easy and quick coupling system built-in.

Anodized aluminum structure.

Tachometer.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

Dimensions: 800 x 500 x 600 mm. approx. (31.49 x 19.68 x 23.62 inches approx.).  Weight: 50 Kg. approx. (110 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Flow calculation.

2.- Obtaining of the M (n, Ha), N(n, Ha),  (n, Ha) curves.

3.- Obtaining of the M (n, Q), Nm (n, Q),  (n, Q) curves.

4.- Adimensionalization.

Optional

LIFLUBA/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.

BDAS. Basic Data Acquisition System and Sensors:

- BDAS/BFA:

Base Module:

Anodized aluminum frame and panel made of painted steel.

Main elements made of stainless steel.

This unit has wheels to facilitate its mobility.

Sensors (only the sensors that correspond according to the purchased module/s will be included):

Two pressure sensors, range: 0 – 100 PSI (0 – 7 bars).

Two differential pressure sensors: they measure displacement through two manometers with a range of up to 1 meter, scale: 0.1 mm.

Flow sensor for high working flows, range: 5 – 150  l/min.

Flow sensor for low working flows, range: 1.5 – 30  l/min.

Piezoresistive force sensor. It measures from 0 kg to 1.5 kg and converts the value into Newton.

Force sensor to calculate the braking torque of turbines. It measures from 0 kg to 2 kg and converts the value into Newton.

Optical speed sensor to measure the speed of turbines.

Measurements of the speed and torque of the pumps.

Dimensions: 300 x 550 x 1200 mm approx. (11.81 x 21.65 x 47.24 inches approx.).  Weight: 10 kg approx. (22 pounds approx.).

Data Acquisition System:

Data Acquisition Electronic Box, with connectors for the different sensors.

PCI Express data acquisition board (National Instruments) that is lodged in a slot of the computer. PCI Express bus.

Data acquisition software.

This system enables:

To represent the system responses curves in real time.

To record all the measurement values and results in a file.

To plot the characteristic curves.

To calibrate the sensors that take part in the process.

Dimensions: 490 x 330 x 310 mm approx. (19.29 x 12.99 x 12.20 inches approx.) Weight: 10 kg approx. (22 pounds approx.).

LIFLUBA/BDAS-BFI: Specific Software and accessories for each module type “FME”:.

These accessories are used to adapt each module in order to operate them and connect them to the Base Module (BDAS/BFA) easily.