Tender Specifications (for main items)

1 CFC. Unit:

Anodized aluminum frame and panels made of painted steel.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Channel of rectangular section with transparent walls, formed by methacrylate transparent sections.

Available versions to choose:

- CFC80/2. Computer Controlled Flow Channel (section: 80 x 300 mm), length: 2.5 m.

- CFC80/5. Computer Controlled Flow Channel (section: 80 x 300 mm), length: 5 m.

The channel is assembled on supports, with a system to control the inclination of the channel. Channel slope adjustable.

Inlet tank (capacity: 38  l.), with stilling of flow and with drain valve.

Reception tank (capacity: 38  l.), with drain valve.

Pipes.

Orifice plate flow sensor.

Venturi type flow sensor.

FME00/B. Basic Hydraulic Feed System:

Storage tank (capacity: 140  l. approx.).

Impulsion pump, with speed regulation, computer controlled:

Single-phase, 220V/50Hz or 110V/60Hz.

0.37 KW.

2800 r.p.m.

30 - 80  l./min. at 20.1 - 12.8 m.

Safety switch ON/OFF.

Flow control valve.

Flow sensor.

Differential pressure sensor (included in CFTPC).

Displacement sensor (included in CFRMC).

The complete unit includes as well:

Advanced Real-Time SCADA.

Open Control + Multicontrol + Real-Time Control.

Specialized EDIBON Control Software based on LabVIEW.

National Instruments Data Acquisition board (250 KS/s, kilo samples per second).

Calibration exercises, which are included, teach the user how to calibrate a sensor and the importance of checking the accuracy of the sensors before taking measurements.

Projector and/or electronic whiteboard compatibility allows the unit to be explained and demonstrated to an entire class at one time.

Capable of doing applied research, real industrial simulation, training courses, etc.

Remote operation and control by the user and remote control for EDIBON technical support, are always included.

Totally safe, utilizing 4 safety systems (Mechanical, Electrical, Electronic & Software).

Designed and manufactured under several quality standards.

Optional ICAI software to create, edit and carry out practical exercises, tests, exams, calculations, etc.

Apart from monitoring user's knowledge and progress reached.

This unit has been designed for future expansion and integration. A common expansion is the EDIBON Scada-Net (ESN) System which enables multiple students to simultaneously operate many units in a network.

Models and Accessories required:

- CFRMC. Level Gauge for Measurement of the Water Height (Hook and Point Gauge).

- CFTPC. Pitot Tube.

Optional Models and Accessories:

- CFPR. Adjustable Undershot Weir.

- CFCVR. Vertical Flat Gate and Radial Gate.

- CFVD. Sharp Crested Weir.

- CFVG. Broad Crested Weir.

- CFVC. Crump Weir.

- CFPV. Weirs.

- CFVOTP. Ogee Type Weir with Pressure Measurement.

- CFMDE. Energy Dissipation Accesories.

- CFSDL. Syphon Spillway.

- CFSDS. Air Regulated Syphon.

- CFMPL. Model of Plain Beach.

- CFMU. Model of Sill.

- CFMP. Models of Piers.

- CFMA. Models of Culvert.

- CFCA. Culvert Fitting.

- CFPVI. Set of Vibrating Piles.

- CFPLR. Artificial Roughened Bed.

- CFFS. False Floor Sections.

- CFVEN. Venturi Flume.

- CFRMD. Digital Water Level Indicator.

- CFMV. Velocity Meter.

- CFTVC. Differential Pressure Digital Indicator to Measure the Inlet Flow.

- CFAS. Sediment Feeder.

- CFGOC. Computer Controlled Wave Generator.

- CFTS. Sediment Trap.

2 CFC/CIB. Control Interface Box:

The Control Interface Box is part of the SCADA system.

Control interface box with process diagram in the front panel.

The unit control elements are permanently computer controlled.

Simultaneous visualization in the computer of all parameters involved in the process.

Calibration of all sensors involved in the process.

Real time curves representation about system responses.

All the actuators’ values can be changed at any time from the keyboard allowing the analysis about curves and responses of the whole process.

Shield and filtered signals to avoid external interferences.

Real time computer control with flexibility of modifications from the computer keyboard of the parameters, at any moment during the process.

Real time computer control for parameters involved in the process simultaneously.

Open control allowing modifications, at any moment and in real time, of parameters involved in the process simultaneously.

Three safety levels, one mechanical in the unit, another electronic in the control interface and the third one in the control software.

3 DAB. Data Acquisition Board:

The Data Acquisition board is part of the SCADA system.

PCI Express Data acquisition board (National Instruments) to be placed in a computer slot.

Analog input: Channels= 16 single-ended or 8 differential. Resolution=16 bits, 1 in 65536. Sampling rate up to: 250 KS/s (kilo samples per second).

Analog output: Channels=2. Resolution=16 bits, 1 in 65536.

Digital Input/Output: Channels=24 inputs/outputs.

4 CFC/CCSOF. Computer Control +Data Acquisition+Data Management Software:

The three softwares are part of the SCADA system.

Compatible with the industry standards.

Flexible, open and multicontrol software, developed with actual windows graphic systems, acting simultaneously on all process parameters.

Management, processing, comparison and storage of data.

Sampling velocity up to 250 KS/s (kilo samples per second).

Calibration system for the sensors involved in the process.

It allows the registration of the alarms state and the graphic representation in real time.

Open software, allowing the teacher to modify texts, instructions. Teacher’s and student’s passwords to facilitate the teacher’s control on the student, and allowing the access to different work levels.

This unit allows the 30 students of the classroom to visualize simultaneously all the results and the manipulation of the unit, during the process, by using a projector or an electronic whiteboard.

5 Cables and Accessories, for normal operation.

6 Manuals:

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

*References 1 to 6 are the main items: CFC + CFC/CIB + DAB + CFC/CCSOF + Cables and Accessories + Manuals are included in the minimum supply for enabling normal and full operation.

EXERCISES AND PRACTICAL POSSIBILITIES TO BE DONE WITH THE MAIN ITEMS

1.-  Measurement of the water height and the velocity along the channel.

2.-  Measurement of the flow with weirs of thin wall.

3.-  Measurement of flow with changes in the channel section.

4.-  Measurement of flow using Venturi flume.

5.-  Control of the flow by gates.

6.-  Level control using syphons.

7.-  Flow on overflow dams.

8.-  Flow among the pillars of a bridge.

9.-  Connection of a channel to a culvert.

10.-Characterization of the hydraulic jump.

11.-Profiles of the water free surface.

12.-Investigation of flow and supercritical flow states.

13.-Measurement of water levels.

14.-Discharge processes on an underwater weir.

15.-Amount of energy in flows in open channels.

16.-Function of a syphon weir.

17.-Flow rate and drain coefficients of a syphon weir.

18.-Pipe flows.

19.-Comparison of overflow and syphon weirs.

20.-Observation of the throw of the water.

21.-Generation of different flow states by damming the down-stream water.

22.-Observation of the flow under an undershot weir:

- Observation of hydraulic motion on discharge.

23.-Relationship between dam height and discharge.

24.-Observation of discharges under a radial gate:

- Observation of hydraulic motion on discharge.

25.-Hydrostatic pressure on a weir.

26.-Investigations on waves.

27.-Behaviour of structures in rough sea.

28.-Applying and understanding Manning’s formula.

29.-Understanding sub- and super-critical flow.

30.-Learning how to apply force-momentum and steady flow energy equations to simple flow situations.

31.-Investigation of the transition from running to shooting flow.

Additional practical possibilities:

32.-Sensors calibration.

33.-Filling of the Pitot tube.

34.-Filling of the venturi meter with analog output.

35.-Calculation of water flow.

36.-Use of level gauge for measurement of the water height.

Other possibilities to be done with this Unit:

37.-Many students view results simultaneously.

To view all results in real time in the classroom by means of a projector or an electronic whiteboard.

38.-Open Control, Multicontrol and Real Time Control.

This unit allows intrinsically and/or extrinsically to change the span, gains; proportional, integral, derivate parameters; etc, in real time.

39.-The Computer Control System with SCADA allows a real industrial simulation.

40.-This unit is totally safe as uses mechanical, electrical and electronic, and software safety devices.

41.-This unit can be used for doing applied research.

42.-This unit can be used for giving training courses to Industries even to other Technical Education Institutions.

43.-Control of the CFC unit process through the control interface box without the computer.

44.-Visualization of all the sensors values used in the CFC unit process.

- By using PLC-PI additional 19 more exercises can be done.

- Several other exercises can be done and designed by the user.

Tender Specifications (for optional items)

a) Industrial configuration

7 PLC. Industrial Control using PLC (it includes PLC-PI Module plus PLC-SOF Control Software):

-PLC-PI. PLC Module:

Metallic box.

Circuit diagram in the module front panel.

Digital inputs (X) and Digital outputs (Y) block: 16 Digital inputs. 14 Digital outputs.

Analog inputs block: 16 Analog inputs.

Analog outputs block: 4 Analog outputs.

Touch screen.

Panasonic PLC:

High-speed scan of 0.32 µsec. Program capacity of 32 Ksteps. High-speed counter. Multi-point PID control.

Digital inputs/outputs and analog inputs/outputs Panasonic modules.

-CFC/PLC-SOF. PLC Control Software:

For this particular unit, always included with PLC supply.

Practices to be done with PLC-PI:

1.-  Control of the particular unit process through the control interface box without the computer.

2.-  Visualization of all the sensors values used in the particular unit process.

3.-  Calibration of all sensors included in the particular unit process.

4.-  Hand on of all the actuators involved in the particular unit process.

5.-  Realization of different experiments, in automatic way, without having in front the particular unit. (These experiments can be decided previously).

6.- Simulation of outside actions, in the cases do not exist hardware elements. (Example: test of complementary tanks, complementary industrialenvironment to the process to be studied, etc).

7.-  PLC hardware general use.

8.-  PLC process application for the particular unit.

9.-  PLC structure.

10.-PLC inputs and outputs configuration.

11.-PLC configuration possibilities.

12.-PLC program languages.

13.-PLC different programming standard languages (ladder diagram (LD), structured text (ST), instructions list (IL), sequential function chart (SFC), function block diagram (FBD)).

14.-New configuration and development of new process.

15.-Hand on an established process.

16.-To visualize and see the results and to make comparisons with the particular unit process.

17.-Possibility of creating new process in relation with the particular unit.

18.-PLC Programming Exercises.

19.-Own PLC applications in accordance with teacher and student requirements.

b) Technical and Vocational Education configuration

8 CFC/ICAI. Interactive Computer Aided Instruction Software System.

This complete software package consists of an Instructor Software (EDIBON Classroom Manager -ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft -ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

-ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc...so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

User Data Base Management.

Administration and assignment of Workgroup, Task and Training sessions.

Creation and Integration of Practical Exercises and Multimedia Resources.

Custom Design of Evaluation Methods.

Creation and assignment of Formulas & Equations.

Equation System Solver Engine.

Updatable Contents.

Report generation, User Progression Monitoring and Statistics.

-ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired technical knowledge.

Innovative features:

Student Log-In & Self-Registration.

Existing Tasks checking & Monitoring.

Default contents & scheduled tasks available to be used from the first session.

Practical Exercises accomplishment by following the Manual provided by EDIBON.

Evaluation Methods to prove your knowledge and progression.

Test self-correction.

Calculations computing and plotting.

Equation System Solver Engine.

User Monitoring Learning & Printable Reports.

Multimedia-Supported auxiliary resources.

9 CFC/FSS. Faults Simulation System.

Faults Simulation System (FSS) is a Software package that simulates several faults in any EDIBON Computer Controlled Unit.

The "FAULTS" mode consists in causing several faults in the unit normal operation. The student must find them and solve them.

There are several kinds of faults that can be grouped in the following sections:

Faults affecting the sensors measurement:

- An incorrect calibration is applied to them.

- Non-linearity.

Faults affecting the actuators:

- Actuators channels interchange at any time during the program execution.

- Response reduction of an actuator.

Faults in the controls execution:

- Inversion of the performance in ON/OFF controls.

- Reduction or increase of the calculated total response.

- The action of some controls is annulled.

On/off faults:

- Several on/off faults can be included.

c) Multipost Expansions options

10 Mini ESN. EDIBON Mini Scada-Net System.

EDIBON Mini Scada-Net System allows up to 30 students to work with a Teaching Unit in any laboratory, simultaneously.

The Mini ESN system consists of the adaptation of any EDIBON Computer Controlled Unit with SCADA integrated in a local network.

This system allows to view/control the unit remotely, from any computer integrated in the local net (in the classroom), through the main computer connected to the unit.

Main characteristics:

- It allows up to 30 students to work simultaneously with the EDIBON Computer Controlled Unit with SCADA, connected in a local net.

- Open Control + Multicontrol + Real Time Control + Multi Student Post.

- Instructor controls and explains to all students at the same time.

- Any user/student can work doing "real time" control/multicontrol and visualisation.

- Instructor can see in the computer what any user/student is doing in the unit.

- Continuous communication between the instructor and all the users/students connected.

Main advantages:

- It allows an easier and quicker understanding.

- This system allows you can save time and cost.

- Future expansions with more EDIBON Units.

The system basically will consist of:

This system is used with a Computer Controlled Unit.

- Instructor’s computer.

- Students’ computers.

- Local Network.

- Unit-Control Interface adaptation.

- Unit Software adaptation.

- Webcam.

- Mini ESN Software to control the whole system.

- Cables and accessories required for a normal operation.