Tender Specifications

The "TELT" unit consists of:

Anodized aluminum frame and panels made of painted steel.

The unit includes wheels to facilitate its mobility.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Ten hose connections with quick-release couplings.

Four closable intakes to pour the contaminants.

Two cold water intakes.

Toilet tank.

Safety elements of the system:

Bleed valve.

Hot water safety valve: up to 70 ºC.

Thermostatic automatic mixer:

Minimum flow rate: 300  l/h.

Maximum pressure: 1 – 5 bar.

Safety stop for a maximum temperature of 70 ºC.

Two-handle mixer.

Hot water boiler simulator.

Sewage line.

Pipes:

Copper.

Sections made of PMMA for inside visualization.

Manuals: This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

Dimensions: 2000 x 300 x 1300 mm approx. (78.74 x 11.81 x 51.18 inches approx.)   Weight: 100 Kg approx. (220 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.-  Study of pipes flushing techniques according to the DIN 1988/83/CE standard.

2.-  Use of original components from plumbing systems.

3.-  Understanding the operation of the main elements of a house sanitation system.

4.-  Limitations of pipes flushing process.

Optional

TELT/ICAI. Interactive Computer Aided Instruction Software System:

This complete software package consists of an Instructor Software (INS/SOF) totally integrated with the Student Software (TELT/SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.