Tender Specifications

Bench-top unit.

Anodized aluminum frame and panels made of painted steel.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Condenser: finned exchanger.

Evaporator.

Absorber.

Absorption tank.

Compartment with freezing zone and cooling zone with one temperature sensor in each zone.

Solenoid valve.

Generator or heater, possibility to select the heat source between an heating element or LPG:

To work with LPG:

Burner and piezoelectric igniter: to ignite the flame of fuel.

Pressure regulator with manometer (range 0 – 3 bar) to regulate the LPG inlet pressure to the generator.

To work with an electrical heating element:

Electrical heater: 230 V, 125 W.

Control of power.

Instrumentation:

Ten temperature sensors distributed at key points of the unit.

Sensors for the power by the electrical heating element.

The unit includes all the safety measures required for a safe operation:

The unit is perfectly watertight to avoid ammonia leakages.

Safety pushbutton.

Electronic console:

Metallic box.

Connectors for the temperature sensors.

Sensor selector for temperature sensors.

Digital display for temperature sensors.

Heating element switch.

Heating element regulator.

Digital display for the power of the heating element.

Main switch.

Cables and Accessories, for normal operation.

Manuals: This unit is supplied whit the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

Unit: -Dimensions: 700 x 700 x 700 mm approx. (27.55 x 27.55 x 27.55 inches approx.) -Weight: 70 kg approx. (154.32 pounds approx.)

Electronic console: -Dimensions: 490 x 330 x 310 mm approx. (19.29 x 12.99 x 12.2 inches approx.) -Weight: 10 kg approx. (22 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.-  Demonstration of the refrigeration process.

2.-  Demonstration of the vapour absorption refrigeration cycle and visualization of the most important processes.

3.-  Familiarisation with the individual components of the absorption refrigeration unit.

4.-  Operation of the gas absorption refrigeration unit using either an electric element or LPG as the heat source.

5.-  Comparison of the refrigeration temperature obtained using LPG or an electrical element as heat source.

6.-  Measurement of the electrical power.

7.-  Influence of the electrical power in the refrigeration temperature obtained.

8.- Measurement of the temperature in the main points of the absorption refrigeration process.

Optional

TRAB/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.