TENDER SPECIFICATIONS (for main items)

1  PLGC. Unit:

Anodized aluminium structure and panels in painted steel. Main metallic elements in stainless steel.

Diagram in the front panel with similar distribution to the elements in the real unit.

Packed Column:

Column formed by a glass cylindrical tube of 1200 mm. of length and an internal diameter of 100 mm.

The column is filled with Raschig rings with a 8 mm. diameter, supported on a PVC plate.

Rectangular tank formed by the reaction system and the settler, with a methacrylate structure. Height: 300 mm. and length: 440 mm.

Cylindrical feed water tank. Height: 300 mm. and diameter: 300 mm.

Pump, computer controlled, to supply water to the column:

STD Point: 4m-20   l./min, 2700/3100 rpm.

Pump, computer controlled, to supply the flocculant to the reaction tank.

Solids dosing system to introduce the powders in the air line.

Compressor (blower), computer controlled:

2700rpm, 0.25kW.

Stirrer, computer controlled, to mix the water going out the column with the flocculant.

2 two-way valves to regulate the flow, placed before the air and water flowmeters.

Ball valve for extracting the sludges.

Sensors:

Flow sensor to determine the air flow, range: 0 - 4m3/h.

Flow sensor to measure the water flow, range: 0.2-6  l./min.

Two flowmeters.

Differential pressure sensor at the column.( 1psi).

PH sensor.

The unit has wheels for its mobility.

The complete unit includes as well:

Advanced Real-Time SCADA.

Open Control + Multicontrol + Real-Time Control.

Specialized EDIBON Control Software based on Labview.

National Instruments Data Acquisition board (250 KS/s , kilo samples per second).

Calibration exercises, which are included, teach the user how to calibrate a sensor and the importance of checking the accuracy of the sensors before taking measurements.

Projector and/or electronic whiteboard compatibility allows the unit to be explained and demonstrated to an entire class at one time.

Capable of doing applied research, real industrial simulation, training courses, etc.

Remote operation and control by the user and remote control for EDIBON technical support, are always included.

Totally safe, utilizing 4 safety systems (Mechanical, Electrical, Electronic & Software).

Designed and manufactured under several quality standards.

Optional CAL software helps the user perform calculations and comprehend the results.

This unit has been designed for future expansion and integration. A common expansion is the EDIBON Scada-Net (ESN) System which enables multiple students to simultaneously operate many units in a network.

2  PLGC/CIB. Control Interface Box:

The Control Interface Box is part of the SCADA system. Control interface box with process diagram in the front panel.

The unit control elements are permanently computer controlled.

Simultaneous visualization in the computer of all parameters involved in the process.

Calibration of all sensors involved in the process.

Real time curves representation about system responses.

All the actuators’ values can be changed at any time from the keyboard allowing the analysis about curves and responses of the whole process.

Shield and filtered signals to avoid external interferences.

Real time computer control with flexibility of modifications from the computer keyboard of the parameters, at any moment during the process.

Real time computer control for parameters involved in the process simultaneously.

Open control allowing modifications, at any moment and in real time, of parameters involved in the process simultaneously.

Three safety levels, one mechanical in the unit, another electronic in the control interface and the third one in the control software.

3  DAB. Data Acquisition Board:

The Data Acquisition board is part of the SCADA system. PCI Data acquisition board (National Instruments) to be placed in a computer slot.

Analog input: Channels= 16 single-ended or 8 differential. Resolution=16 bits, 1 in 65536. Sampling rate up to: 250 KS/s  (Kilo samples per second).

Analog output: Channels=2.  Resolution=16 bits, 1 in 65536.

Digital Input/Output: Channels=24 inputs/outputs.

4  PLGC/CCSOF. Computer Control +Data Acquisition+Data Management Software:

The three softwares are part of the SCADA system. Compatible with the industry standards.

Flexible, open and multicontrol software, developed with actual windows graphic systems, acting simultaneously on all process parameters.

Management, processing, comparison and storage of data. Sampling velocity up to 250 KS/s (Kilo samples per second).

Calibration system for the sensors involved in the process.

It allows the registration of the alarms state and the graphic representation in real time.

Open software, allowing the teacher to modify texts, instructions.

Teacher’s and student’s passwords to facilitate the teacher’s control on the student, and allowing the access to different work levels.

This unit allows the 30 students of the classroom to visualize simultaneously all the results and the manipulation of the unit, during the process, by using a projector or an electronic whiteboard.

5  Cables and Accessories, for normal operation.

6 Manuals: This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

* References 1 to 6 are the main items: PLGC + PLGC/CIB + DAB + PLGC/CCSOF + Cables and Accessories + Manuals are included in the minimum supply for enabling normal and full operation

Exercises and Practical Possibilities to be done with Main Items

1.-	Familiarization with a gas washing plant.

2.-	Study of the water flow influence on the gas washing plant efficiency.

3.-	Study of the air flow influence on the gas washing plant efficiency.

4.-	Study of the introduced dust mass influence on the gas washing plant efficiency.

5.-	Determination of the gas washing plant efficiency related to the quantity of solids introduced into the air current with the quantity of sedimented solids.

6.-	Best quantity of floculant to produce the dust particles precipitation.

7.-	Determination of the best pH for the solid particles sedimentation which go together with the water.

8.-	Study of the best pH to obtain biggest purity in settled water.

9.-	Study of the floculant presence influence on the pH variation in the reaction-settling tank.

10.-	Pressure losses in the gas washing plant.

Additional practical possibilities:

11.-	Sensors calibration.

Other possibilities to be done with this Unit:

12.-	Many students view results simultaneously.

To view all results in real time in the classroom by means of a projector or an electronic whiteboard.

13.-	Open Control, Multicontrol and Real Time Control.

This unit allows intrinsically and/or extrinsically to change the span, gains; proportional, integral, derivate parameters; etc in real time.

14.-	The Computer Control System with SCADA allows a real industrial simulation.

15.-	This unit is totally safe as uses mechanical, electrical and electronic, and software safety devices.

16.-	This unit can be used for doing applied research.

17.-	This unit can be used for giving training courses to Industries even to other Technical Education Institutions.

18.-	Control of the PLGC unit process through the control interface box without the computer.

19.-	Visualization of all the sensors values used in the PLGC unit process.

- By using PLC-PI additional 19 more exercises can be done.

- Several other exercises can be done and designed by the user.

TENDER SPECIFICATIONS (for optional items)

a) Industrial configuration

7  PLC. Industrial Control using PLC (it includes PLC-PI Module plus PLC-SOF Control Software):

-PLC-PI. PLC Module:

Metallic box.

Circuit diagram in the module front panel.

Digital inputs(X) and Digital outputs (Y) block: 16 Digital inputs. 14 Digital outputs.

Analog inputs block: 16 Analog inputs.

Analog outputs block: 4 Analog outputs. Touch screen.

Panasonic PLC:

High-speed scan of 0.32 sec.  Program capacity of 32 Ksteps.  High-speed counter.  Multi-point PID control.

Digital inputs/outputs and analog inputs/outputs Panasonic modules.

-PLGC/PLC-SOF. PLC Control Software.

For this particular unit, always included with PLC supply.

Practices to be done with PLC-PI:

1.-	Control of the PLGC unit process through the control interface box without the computer.

2.-	Visualization of all the sensors values used in the PLGC unit process.

3.-	Calibration of all sensors included in the PLGC unit process.

4.-	Hand on of all the actuators involved in the PLGC unit process.

5.-	Realization of different experiments, in automatic way, without having in front the unit. (This experiment can be decided previously).

6.-	Simulation of outside actions, in the cases hardware elements do not exist. (Example: test of complementary tanks, complementary industrial environment to the process to be studied, etc).

7.-	PLC hardware general use and manipulation.

8.-	PLC process application for PLGC unit.

9.-	PLC structure.

10.-	PLC inputs and outputs configuration.

11.-	PLC configuration possibilities.

12.-	PLC programming languages.

13.-	PLC different programming standard languages.

14.-	New configuration and development of new process.

15.-	Hand on an established process.

16.-	To visualize and see the results and to make comparisons with the PLGC unit process.

17.-	Possibility of creating new process in relation with the PLGC unit.

18.-	PLC Programming exercises.

19.-	Own PLC applications in accordance with teacher and student requirements.

b) Technical and Vocational Education configuration

8  PLGC/CAI. Computer Aided Instruction Software System.

This complete package consists on an Instructor Software (INS/ SOF) totally integrated with the Student Software (PLGC/SOF).

-INS/SOF. Classroom Management Software (Instructor Software):

The Instructor can:

Organize Students by Classes and Groups.

Create easily new entries or delete them.

Create data bases with student information.

Analyze results and make statistical comparisons.

Print reports.

Develop own examinations.

Detect student’s progress and difficulties.

-PLGC/SOF. Computer Aided Instruction Software (Student Software):

It explains how to use the unit, run the experiments and what to do at any moment.

This Software contains:

Theory.

Exercises.

Guided Practices.

Exams.

9  PLGC/FSS. Faults Simulation System.

Faults Simulation System (FSS) is a Software package that simulates several faults in any EDIBON Computer Controlled Unit.

The "FAULTS" mode consists on causing several faults in the unit normal operation. The student must find them and solve them.

There are several kinds of faults that can be grouped in the following sections:

Faults affecting the sensors measurement:

- An incorrect calibration is applied to them.

- Non-linearity.

Faults affecting the actuators:

- Actuators channels interchange at any time during the program execution.

- Response reduction of an actuator.

Faults in the controls execution:

- Inversion of the performance in ON/OFF controls.

- Reduction or increase of the calculated total response.

- The action of some controls is annulled.

On/off faults:

- Several on/off faults can be included.

c) Higher Education and/or Technical and Vocational Education configuration

10  PLGC/CAL. Computer Aided Learning Software (Results Calculation and Analysis).

This Computer Aided Learning Software (CAL) is a Windows based software, simple and very easy to use.

CAL is a class assistant that helps in making the necessary calculations to extract the right conclusions from data obtained during the experimental practices.

CAL will perform the calculations.

CAL computes the value of all the variables involved.

It allows to plot and print the results. Between the plotting options, any variable can be represented against any other.

Different plotting displays.

It has a wide range of information, such as constant values, unit conversion factors and integral and derivative tables available.

d) Multipost Expansions options

11  Mini ESN. EDIBON Mini Scada-Net System.

EDIBON Mini Scada-Net System allows up to 30 students to work with a Teaching Unit in any laboratory, simultaneously.

The Mini ESN system consists on the adaptation of any EDIBON Computer Controlled Unit with SCADA integrated in a local network.

This system allows to view/control the unit remotely, from any computer integrated in the local net (in the classroom), through the main computer connected to the unit.

Main characteristics:

-It allows up to 30 students to work simultaneously with the EDIBON Computer Controlled Unit with SCADA, connected in a local net.

-Open Control + Multicontrol + Real Time Control + Multi Student Post.

-Instructor controls and explains to all students at the same time.

-Any user/student can work doing "real time" control/multicontrol and visualisation.

-Instructor can see in the computer what any user/student is doing in the unit.

-Continuous communication between the instructor and all the users/students connected.

Main advantages:

-It allows an easier and quicker understanding.

-This system allows you can safe time and cost.

-Future expansions with more EDIBON Units.

The system basically will consist of:

This system is used with a Computer Controlled Unit.

-Instructor’s computer.

-Students’ computers.

-Local Network.

-Unit-Control Interface adaptation.

-Unit Software adaptation.

-Webcam.

-Mini ESN Software to control the whole system.

-Cables and accessories required for a normal operation.