TENDER SPECIFICATIONS

Bench-top unit.

Anodized aluminum frame and panels made of painted steel.

Main metal elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

An output up to 125  l per hour can be obtained, depending on the degree of separation.

Supply tank made of anodized aluminum:

Capacity: 10  l.

It can be filled in continuously.

Funnel with float that regulates the milk emptying in the separator.

Manifold with outlets for the cream and the skimmed milk.

Separator bowl with 19 discs (separating plates).

Motor with speed control.

Included accessories:

Stopwatch.

Two beakers to collect the product.

Allen key.

Brushes.

The unit can be disassembled for easy cleaning.

Control panel that includes:

Switch to start the motor.

Speed controller of the motor.

Display that indicates the speed measurements of the motor.

ON/OFF switch.

The complete unit includes as well:

Cables and Accessories, for normal operation.

Manuals: This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

-Dimensions: 500 x 500 x 500 mm approx. (19.68 x 19.68 x 19.68 inches approx.). -Weight:25 Kg approx. (55 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.-  Study of the separation of different density liquids.

2.-  Demonstration of the effect of the temperature in the separating process.

3.-  Demonstration of the effect of the speed in the separating process.

4.-  Production of different types of cream by using milks with different greasy matter contents, 11% to 55%.

5.-  Production of a range of skimmed milks with different contents of greasy matter.

6.-  To show the importance of cleanness and hygiene in food processing.

7.-  Understanding the operation mode of this type of centrifuge, using instructive diagrams and ease of strip down the component parts.

Optional

DSN/ICAI. Interactive Computer Aided Instruction Software System:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.