Tender Specifications

Bench-top unit with adjustable legs.

Anodized aluminum structure and panels in painted steel.

The MBI unit mainly consists of:

Piston and guide with graduated scale: piston stroke: 70 mm.

Connecting rod made of stainless steel.

Shaft of the system and counterweight made of stainless steel.

Measuring disc.

Beam balance arm.

Set of weights to add to the piston:

Mass of 1 kg, 1 unit.

Mass of 0.5 kg, 2 units.

In order to carry out some of the practices with MBI unit, a “B” type Set of weights is required.

Manuals: This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance and Practices manual.

Dimensions: 400 x 750 x 500 mm. approx.  (15.74 x 29.52 x 19.68 inches approx.).   Weight: 20 Kg. approx. (44 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Determination of the crank effort.

2.- Balancing of the crankshaft.

3.- Measurement of the piston displacement.

4.- Experimental relationship between piston displacement and crank angle.

5.- Theoretical relationship between piston displacement and crank angle.

6.- Comparison of the piston displacement value theoretically and experimentally.

7.- Experimental relationship between turning moment and crankshaft angle.

8.- Theoretical relationship between turning moment and crankshaft angle.

Optional

MBI/ICAI. Interactive Computer Aided Instruction Software System:

this complete software package consists of an Instructor Software (EDIBON Classroom Manager -ECM) totally integrated with the Student Software (EDIBON Student Labsoft -ESL). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

-ECM. EDIBON Classroom Manager (Instructor Software).

ECM is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc...so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

User Data Base Management.

Administration and assignment of Workgroup, Task and Training sessions.

Creation and Integration of Practical Exercises and Multimedia Resources.

Custom Design of Evaluation Methods.

Creation and assignment of Formulas & Equations.

Equation System Solver Engine.

Updatable Contents.

Report generation, User Progression Monitoring and Statistics.

-ESL. EDIBON Student Labsoft (Student Software).

ESL is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired technical knowledge.

Innovative features:

Student Log-In & Self-Registration.

Existing Tasks checking & Monitoring.

Default contents & scheduled tasks available to be used from the first session.

Practical Exercises accomplishment by following the Manual provided by EDIBON.

Evaluation Methods to prove your knowledge and progression.

Test self-correction.

Calculations computing and plotting.

Equation System Solver Engine.

User Monitoring Learning & Printable Reports.

Multimedia-Supported auxiliary resources.

BDAS. Basic Data Acquisition System and Sensors.

For being used with mechanical modules.

BDAS is designed to monitor the measurements of each mechanical module from a computer (PC).