Tender Specifications

Bench-top unit with adjustable legs.

Aluminum transportation case.

Anodized aluminum frame and panels made of painted steel.

Simple belt:

Timing Belt Pulleys:

No. of teeth z= 30, 32, 48, 60.

Simple chain:

Chain cog wheels:

No. of teeth z = 20, 30.

Simple straight gears:

Module m = 2; No. of teeth z = 20, 32, 40, 50.

Module m = 1.5; No. of teeth z = 32, 40, 44.

Pair of bevel gears:

Module m = 2; No. of teeth z = 25.

Angle between shafts 90º.

Endless screw gear:

Endless screw: z = 3.

Helical wheel: z = 37.

Rack and pinion gear.

Tool set.

Manuals: This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manual.

Dimensions: Assembled frame: 1100 x 550 x 500 mm approx. (43.30 x 21.65 x 19.68 inches approx.)

Carrying case: 150 x 445 x 310 mm approx. (5.90 x 17.52 x 12.20 inches approx.)

Toolkit: 311 x 168 x 130 mm approx. (12.24 x 6.61 x 5.11 inches approx.)  Weight: 50 Kg approx. (110 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.-  Know the important components of the transmissions.

2.-  Know the essential forms of transmission:

Single-belt transmission.

Simple straight gears.

Conical gears.

Rack and pinion gears.

Endless screw gears.

3.-  Perform calculations in mechanical transmissions.

4.-  Learn the practical assembly of the various possible configurations of mechanical transmissions, and learn to adjust and align.

5.-  Understanding of technical diagrams and familiarization with them.

Optional

MSDA/ICAI. Interactive Computer Aided Instruction Software System:

This complete software package consists of an Instructor Software (INS/SOF) totally integrated with the Student Software (MSDA/SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.