Tender Specifications

Bench-top unit with adjustable legs.

Anodized aluminum structure and panels in painted steel.

The EEFCR unit mainly consists of:

Temperature conditioning box made of methacrylate. Here, the adequate temperature conditions to carry out the experiment are created:

Inside it, the specimen which will be experimented is located.

To reach the temperature, an isotherm bag will be used.

To know the experiment conditions, there is a thermometer at the upper part of the box, range: from -10ºC to 110ºC.

Load arm:

It is a stainless steel beam on which the loads are applied to the specimen.

It pivots on a shaft inserted in the support column.

Tensile stress range: 0-35 N/mm².

Support screw for supporting the load arm before starting the experiment.

Clamps. They are into the space to fasten the specimens.

Dial indicator:

To measure  the elongation of the specimen which is under the load and temperature concrete conditions.

Range: 0-10 mm.

Precision: 0.01 mm.

Allen key to adjust the dial indicator.

Specimens:

Flat type.

Made in different plastic materials: PVC and Polypropylene.

They have a section of 5 mm x 2 mm to make the experiments easier.

In order to carry out some of the practices with EEFCR unit, a “C” Type Set of weights is required.

Manuals: This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

Dimensions: 700 x 350 x 600 mm. approx. (27.56 x 13.78 x 23.62 inches approx.). Weight: 25 Kg. approx. (55.1 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Experimental determination of the behaviour of different plastic materials.

2.- Experimental determination of the creep limit of different plastic materials.

3.- Determination of the temperature effect in the creep behaviour of a plastic material.

4.- Relation of the deformation caused by the creep in function of the time. Phases of the creep.

Optional

EEFCR/ICAI. Interactive Computer Aided Instruction Software System:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager –ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft –ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

-ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc...so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

User Data Base Management.

Administration and assignment of Workgroup, Task and Training sessions.

Creation and Integration of Practical Exercises and Multimedia Resources.

Custom Design of Evaluation Methods.

Creation and assignment of Formulas & Equations.

Equation System Solver Engine.

Updatable Contents.

Report generation, User Progression Monitoring and Statistics.

-ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF  is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired technical knowledge.

Innovative features:

Student Log-In & Self-Registration.

Existing Tasks checking & Monitoring.

Default contents & scheduled tasks available to be used from the first session.

Practical Exercises accomplishment by following the Manual provided by EDIBON.

Evaluation Methods to prove your knowledge and progression.

Test self-correction.

Calculations computing and plotting.

Equation System Solver Engine.

User Monitoring Learning & Printable Reports.

Multimedia-Supported auxiliary resources.

BDAS. Basic Data Acquisition System and Sensors.

For being used with mechanical modules.

BDAS is designed to monitor the measurements of each mechanical module from a computer (PC).