RYC/B allows the user to learn the basics about regulation and control of first and second order systems.

This unit enables to carry a set of practices related with basic regulation and control, through which the user will understand how to characterize first and second order systems and how a PID controller works.

The unit has three modules: reference signals, PID Controller, first order system and second order system module.

## TENDER SPECIFICATIONS

Metallic enclosure, including all the modules and elements.

Power Supply.

Protection fuse.

Block diagrams in the front panel.

Modules:

Reference signals:

Step:	Amplitude:10 V.  Frequency: 0Hz to 1000Hz.

Ramp:  	Amplitude:10 V. Frequency: 0Hz to 1000Hz.

Sine:	Amplitude: 10 V. Frequency: 0Hz to 1000Hz.

PID controller:

P controller:  Kp: 0 to 10.

I controller:   Ti: 1 ms to 10 ms.

D controller: Td: 1 ms to 10 ms.

Systems:

First Order System:

Time constant T: 1ms to 100 ms.

Second Order System:

Damping coefficient  0 to 1.5.

Natural frequency (n): 2* *100 rad/s (100Hz)

Cables and Accessories, for normal operation.

Manuals:

This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

*Dimensions: 490 x 330 x 310 mm. approx.  Weight: 10 Kg. approx.

EXERCISES AND PRACTICAL POSSIBILITIES

Some Practical Possibilities of the Unit:

1.-   Response of a first order system in time domain. (Step-response).

2.-   Response  of  a  first  order  system in time domain. (Ramp-response).

3.-   Response of a first order system in time domain. (Sinusoidal-response).

4.-   Response of a first order system in frequency domain (Sinusoidal-response).

5.-   Response of a second order system in time domain (Step-response).

6.-   Response of a second order system in time domain. (Ramp-response).

7.-   Response of a second order system in time domain. (Sinusoidal-response).

8.-   Response  of  a  second order system in frequency domain (Sinusoidal-response).

9.-   Structure of a PID controller (Proportional-Integrative-Derivative blocks).

10.- PID control of a first order system in open-loop.

11.- PID control of a second order system in open-loop.

12.- PID control of a first order system in closed-loop. (Mathematical tuning).

13.- PID control of a first order system in closed-loop. (Experimental tuning).

14.- PID control of a first order system in closed-loop. (Ziegler-Nichols tuning).

15.- PID control  of  a  second  order  system in closed-loop. (Mathematical tuning).

16.- PID control of a second   order system in closed-loop. (Experimental tuning).

17.- PID control of a second order system in closed-loop. (Ziegler-Nichols tuning).