TENDER SPECIFICATIONS (for main items)

1  TDS. Unit:

Unit mounted in a metallic box.

2 Function Generators modules, each one includes:

Waveform selector: sine, square and triangular.

Frequency potentiometer, range: 20 Hz to 20 KHz.

Amplitude potentiometer, range: ± 10 V.

Duty cycle potentiometer, range: 0% to 100%.

BNC output connector.

Noise Generator module:

White noise generator:

Amplitude potentiometer, range: ± 10 V.

BNC output connector.

Pink noise generator (also called frequency inverter or noise 1/):

Amplitude potentiometer, range: ± 10 V.

BNC output connector.

Microphone Pre-Amplifier module:

It adapts the microphone signal to be analyzed by the data acquisition system or by the speaker.

1/4’’ jack input connector for the microphone.

BNC output connector.

Microphone:

Impedance: 500 .

Frequency response: 70 Hz to 14 KHz.

1/4’’ jack connector.

Power Amplifier module, including:

BNC input connector.

BNC output connector.

Potentiometer to regulate the power amplifier gain.

Speaker module:

Impedance: 8 .

Nominal maximum power: 30 W.

Frequency response: 100 Hz to 13 KHz.

PC input/output module, including:

2 BNC input connectors.

2 BNC output connectors.

SCSI connector to the data acquisition board (to be placed in the computer).

Possibility of working simultaneously with two external signals, facilitating operations that require more than one signal.

Moreover, it is possible to generate different waveforms by the software and send them to the outputs of the unit. These signals can be visualized by an external oscilloscope or be listened by the speaker.

The complete unit includes as well:

Specialized EDIBON Control Software based on Labview.

Projector and/or electronic whiteboard compatibility allows the unit to be explained and demonstrated to an entire class at one time.

Capable of doing applied research, training courses, etc.

Remote operation and control by the user and remote control for EDIBON technical support, are always included.

Totally safe, utilizing 4 safety systems (Mechanical, Electrical, Electronic & Software).

Designed and manufactured under several quality standards.

2  DAB. Data Acquisition Board:

PCI Express Data acquisition board (National Instruments) to be placed in a computer slot.

Analog input: Channels=16 single-ended or 8 differential. Resolution=16 bits, 1 in 65536. Sampling rate up to: 250 KS/s (kilo samples per second).

Analog output: Channels=2. Resolution=16 bits, 1 in 65536.

Digital Input/Output: Channels=24 inputs/outputs.

3  TDS/CCSOF. Computer Control +Data Acquisition+Data Management Software:

Compatible with actual Windows operating systems.

Compatible with the industry standards.

Registration and visualization of signals in an automatic and simultaneous way.

2 signals can be visualized simultaneously.

2 signals can be generated simultaneously.

Totally configurable digital filters: Infinite Impulse Response (IIR) filter and Finite Impulse Response (FIR) filter.

Totally configurable analog filters: Butterworth filter, Chebyshev filter, etc.

Visualization of frequency and time response of the signal before and after filter.

Visualization of each input signal and the signal result of the combination of the two input signals.

Management, processing, comparison and storage of data.

Sampling velocity up to 250 KS/s (kilo samples per second).

4  Cables and Accessories, for normal operation.

5  Manuals: This unit is supplied with the following manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance & Practices Manuals.

* References 1 to 5 are the main items: TDS Unit + DAB + TDS/CCSOF + Cables and Accessories + Manuals are included in the minimum supply for enabling normal and full operation.

EXERCISES AND PRACTICAL POSSIBILITIES TO BE DONE WITH MAIN ITEMS

1.-	Continuous wave form generation, with the possibility of varying the frequency and amplitude of signals, besides of duty cycle.

2.-	Characterization of signals. To analyze the nature of the signals: sine, square, triangular, etc.

3.-	Working simultaneously with two external signals.

4.-	Generating signals directly by the software and send them to the unit outputs and then visualizing or listening by the speaker or an external oscilloscope.

5.-	Signal digitalization, permitting the most suitable sampling time, avoiding “aliasing”.

6.-	Digitalization of signals with the possibility of adjusting the sampling frequency.

7.-	Fast Fourier Transforms (Power Spectrum).

8.-	Addition, subtraction and multiplication of signals.

9.-	Study of “aliasing”.

10.-Study of different noise types:

-White noise.

-1/ƒ noise.

11.-Study and use of filters:

-Possibility of filtration of any signal.

-Reconstruction of signals through the application of filters.

-Digital filters:

Finite Impulse Response (FIR) Filters and Infinite Impulse Response (IIR) Filters.

-Analogue filters:

Butterworth, Chebyshev, etc.

-Possibility to use Bartlett, Hanning, Hamming, Kaiser, Parzen, etc. windows for applying on the signal.

Other possibilities to be done with this Unit:

12.-Many students view results simultaneously.

To view all results in real time in the classroom by means of a projector or an electronic whiteboard.

13.-This unit is totally safe as uses mechanical, electrical and electronic, and software safety devices.

14.-This unit can be used for doing applied research.

15.-This unit can be used for giving training courses to Industries even to other Technical Education Institutions.

- Several other exercises can be done and designed by the user.

TENDER SPECIFICATIONS (for optional items)

a) Technical and Vocational Education configuration

6  TDS/ ICAI. Interactive Computer Aided Instruction Software System.

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

This software is optional and can be used additionally to items (1 to 6).

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical

Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their

knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.