Tender Specifications (for main items)

1 TPTVC/1.5kW. Unit:

Anodized aluminum frame and panels made of painted steel.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Transparent panels to protect the unit that allows the user to observe the operation of the unit.

Cooling pump.

Two inlet tanks.

Two vacuum tanks:

Material: stainless steel.

Pressure gauge.

Safety valve.

Inlet water valve.

Droplet separator.

Turbine:

Until 1.5 kWe.

Inlet conditions: superheated steam: 5.2 bar abs; temperature: 200 - 220 °C.

Outlet conditions: steam: 0.1 bar abs; temperature: 40 – 50 °C.

Condenser:

18 - 30 kW.

Cooling water: temperature: < 30 °C.

Working pressure: 0.1 - 0.2 bar (max.).

Safety pressure switch.

Relief valve in the steam generator: 12 bar.

Float valve.

Working speed control valve:

Limit the speed of the turbine through the computer controlled solenoid valves.

Relief valve in the steam tank.

Power invertor:

230 V - 400 V; 50 HZ, 60Hz.

Instrumentation:

Pressure sensors:

Water inlet to the steam generator.

Steam generator.

Refrigeration water inlet.

Turbine inlet.

Steam turbine outlet.

Condensed water stream.

Second vacuum tank.

Wet steam outlet.

“J” type temperature sensors:

Water inlet to the steam generator.

Steam generator.

Refrigeration water inlet.

Turbine inlet.

Cooling outlet of the turbine.

Steam turbine outlet.

Condensed water stream.

Inlet and outlet of cooling water in the condenser.

Wet steam outlet.

Condensate recovery.

Water inlet to the refrigeration tower.

Water outlet to the refrigeration tower.

Flow sensors:

Refrigeration water inlet.

Steam inlet in the turbine.

Water inlet to the steam generator.

Water inlet to the refrigeration tower.

Speed sensor.

Generated power sensor.

The complete unit includes as well:

Advanced Real-Time SCADA and PID Control.

Open Control + Multicontrol + Real-Time Control.

Specialized EDIBON Control Software based on LabVIEW.

National Instruments Data Acquisition board (250 KS/s, kilo samples per second).

Calibration exercises, which are included, teach the user how to calibrate a sensor and the importance of checking the accuracy of the sensors before taking measurements.

Projector and/or electronic whiteboard compatibility allows the unit to be explained and demonstrated to an entire class at one time.

Capable of doing applied research, real industrial simulation, training courses, etc.

Remote operation and control by the user and remote control for EDIBON technical support, are always included.

Totally safe, utilizing 4 safety systems (Mechanical, Electrical, Electronic & Software).

Designed and manufactured under several quality standards.

Optional ICAI software to create, edit and carry out practical exercises, tests, exams, calculations, etc. Apart from monitoring user's knowledge and progress reached.

This unit has been designed for future expansion and integration. A common expansion is the EDIBON Scada-Net (ESN) System which enables multiple students to simultaneously operate many units in a network.

Optional Models and Accessories: (Not included)

- SFU/. Softener Unit.

- RETU. Refrigeration Tower Unit.

- SGU/B. Steam Generator Unit.

2 TPTVC/1.5kW/CIB. Control Interface Box:

The Control Interface Box is part of the SCADA system.

Control interface box with process diagram in the front panel.

The unit control elements are permanently computer controlled.

Simultaneous visualization in the computer of all parameters involved in the process.

Calibration of all sensors involved in the process.

Real time curves representation about system responses.

All the actuators’ values can be changed at any time from the keyboard allowing the analysis about curves and responses of the whole process.

Shield and filtered signals to avoid external interferences.

Real time PID control with flexibility of modifications from the computer keyboard of the PID parameters, at any moment during the process.

Real time PID control for parameters involved in the process simultaneously.

Proportional control, integral control and derivative control, based on the real PID mathematical formula, by changing the values, at any time, of the three control constants (proportional, integral and derivative constants).

Open control allowing modifications, at any moment and in real time, of parameters involved in the process simultaneously.

Three safety levels, one mechanical in the unit, another electronic in the control interface and the third one in the control software.

3 DAB. Data Acquisition Board:

The Data Acquisition board is part of the SCADA system.

PCI Express Data acquisition board (National Instruments) to be placed in a computer slot.

Analog input: Channels= 16 single-ended or 8 differential. Resolution=16 bits, 1 in 65536. Sampling rate up to: 250 KS/s (kilo samples per second).

Analog output: Channels=2. Resolution=16 bits, 1 in 65536.

Digital Input/Output: Channels=24 inputs/outputs.

4 TPTVC/1.5kW/CCSOF. PID Computer Control +Data Acquisition+Data Management Software:

The three softwares are part of the SCADA system.

Compatible with the industry standards.

Flexible, open and multicontrol software, developed with actual windows graphic systems, acting simultaneously on all process parameters.

Analog and digital PID control. PID menu and set point selection required in the whole work range.

Management, processing, comparison and storage of data.

Sampling velocity up to 250 KS/s (kilo samples per second).

Calibration system for the sensors involved in the process.

It allows the registration of the alarms state and the graphic representation in real time.

Open software, allowing the teacher to modify texts, instructions. Teacher’s and student’s passwords to facilitate the teacher’s control on the student, and allowing the access to different work levels.

This unit allows the 30 students of the classroom to visualize simultaneously all the results and the manipulation of the unit, during the process, by using a projector or an electronic whiteboard.

5 Cables and Accessories, for normal operation.

6 Manuals:

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

*References 1 to 6 are the main items: TPTVC/1.5kW + TPTVC/1.5kW/CIB + DAB + TPTVC/1.5kW/CCSOF + Cables and Accessories + Manuals are included in the minimum supply for enabling normal and full operation.

EXERCISES AND PRACTICAL POSSIBILITIES TO BE DONE WITH THE MAIN ITEMS

Commissioning experiments:

1.-  Study, analysis and test of safety systems.

2.-  Study, analysis and test of measurement systems.

3.-  Study, analysis and test of pressure in the circuit.

4.-  Study of the pressure and temperature control techniques in a steam power plant.

5.-  Study, analysis and test of the water softening unit.

6.-  Study, analysis, operation and test of the steam boiler.

7.-  Study, analysis and test of the steam flow sensor.

8.-  Study, analysis and test of the condenser.

9.-  Study, analysis and test of the refrigeration tower.

10.-Study and analysis of the corresponding maintenance in a steam power plant.

11.-Commissioning of a steam power plant.

12.-Shut down of a steam power plant.

Operating experiments:

13.-Study of the operation of a steam power plant.

14.-Familiarization with a water/steam closed circuit.

15.-Study and understanding of the first and second laws of thermodynamics.

16.-Determination of the steam generator efficiency.

17.-Determination of fuel consumption in function of the steam flow generated.

18.-Determination of the condenser efficiency.

19.-Determination of the amount of heat removed by the condenser.

20.-Determination of the ideal mechanical/thermal efficiency of the turbine.

21.-Determination of the real mechanical/thermal efficiency of the turbine.

22.-Determination of the refrigeration tower efficiency.

23.-Determination of the amount of heat removed by the refrigeration tower.

24.-Determination of the water-steam ratio required by the plant.

25.-Study of the power generated.

26.-Study of the global efficiency of the steam cycle.

27.-Steam flow and measurements range.

Thermodinamic cycle and study of the power generated:

28.-Study, analysis and representation of Rankine cycle for the steam generation plant.

29.-Study, analysis and representation of the generated power in function of the steam pressure, with and without load variation in the generator.

30.-Study, analysis and representation of the steam pressure in function of the revolutions in the steam turbine, with and without load variation in the generator.

31.-Study, analysis and representation of the generated power in function of the type of intake to the turbine, with constant working pressure, with and without load variation in the generator.

32.-Study, analysis and representation of the generated power in function of the vacuum pressure at the turbine outlet, with and without load variation in the generator.

33.-Study, analysis and representation of the vacuum pressure at the turbine outlet in function of the revolutions of the turbine, with and without load variation in the generator.

Parameters of the power generation:

34.-Study of the relation between the power delivered to the grid and the steam flow.

35.-Study of the relation between the power delivered to the grid and the steam pressure.

36.-Study of the relation between the power delivered to the grid and the vacuum pressure at the turbine outlet.

37.-Study of the relation of the active power of the generator in function of the steam flow in an isolated circuit (island mode).

38.-Study of the relation of the active power of the generator in function of the steam pressure in an isolated circuit (island mode).

39.-Study of the relation of the active power generation of the generator in function of the vacuum pressure at the turbine outlet in an isolated circuit (island mode).

40.-Study of the turbine fluctuation and the generator when suffering a sudden change in the power demand.

41.-Study of the synchronization procedure of turbine-generator group with the electrical grid through a grid inverter.

42.-Study of the consequences suffered when the generator is suddenly uncoupled from the electrical grid. Checking the safety systems of the power plant.

Additional practical possibilities:

43.-Study of heat losses in pipes.

44.-Study of the most important parameters in a steam power plant.

45.-Study of the steam generator efficiency in function of fuel used.

46.-Sensors calibration.

Other possibilities to be done with this Unit:

47.-Many students view results simultaneously.

To view all results in real time in the classroom by means of a projector or an electronic whiteboard.

48.-Open Control, Multicontrol and Real Time Control.

This unit allows intrinsically and/or extrinsically to change the span, gains; proportional, integral, derivative parameters; etc, in real time.

49.-The Computer Control System with SCADA and PID Control allow a real industrial simulation.

50.-This unit is totally safe as uses mechanical, electrical and electronic, and software safety devices.

51.-This unit can be used for doing applied research.

52.-This unit can be used for giving training courses to Industries even to other Technical Education Institutions.

53.-Control of the TPTVC/1.5kW unit process through the control interface box without the computer.

54.-Visualization of all the sensors values used in the TPTVC/1.5kW unit process.

- Several other exercises can be done and designed by the user.

Tender Specifications (for optional items)

a) Technical and Vocational Education configuration

8 TPTVC/1.5kW/ICAI. Interactive Computer Aided Instruction Software System.

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc...so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.

9 TPTVC/1.5kW/FSS. Faults Simulation System.

Faults Simulation System (FSS) is a Software package that simulates several faults in any EDIBON Computer Controlled Unit.

The "FAULTS" mode consists in causing several faults in the unit normal operation. The student must find them and solve them.

There are several kinds of faults that can be grouped in the following sections:

Faults affecting the sensors measurement:

- An incorrect calibration is applied to them.

- Non-linearity.

Faults affecting the actuators:

- Actuators channels interchange at any time during the program execution.

- Response reduction of an actuator.

Faults in the controls execution:

- Inversion of the performance in ON/OFF controls.

- Reduction or increase of the calculated total response.

- The action of some controls is annulled.

On/off faults:

- Several on/off faults can be included.

b) Multipost Expansions options

10 MINI ESN. EDIBON Mini Scada-Net System for being used with EDIBON Teaching Units.

MINI ESN. EDIBON Mini Scada-Net System allows up to 30 students to work with a Teaching Unit in any laboratory, simultaneously.

The MINI ESN system consists of the adaptation of any EDIBON Computer Controlled Unit with SCADA and PID Control integrated in a local network.

This system allows to view/control the unit remotely, from any computer integrated in the local net (in the classroom), through the main computer connected to the unit.

Main characteristics:

- It allows up to 30 students to work simultaneously with the EDIBON Computer Controlled Unit with SCADA and PID Control, connected in a local net.

- Open Control + Multicontrol + Real Time Control + Multi Student Post.

- Instructor controls and explains to all students at the same time.

- Any user/student can work doing "real time" control/multicontrol and visualisation.

- Instructor can see in the computer what any user/student is doing in the unit.

- Continuous communication between the instructor and all the users/students connected.

Main advantages:

- It allows an easier and quicker understanding.

- This system allows you can save time and cost.

- Future expansions with more EDIBON Units.

The system basically will consist of:

This system is used with a Computer Controlled Unit.

- Instructor’s computer.

- Students’ computers.

- Local Network.

- Unit-Control Interface adaptation.

- Unit Software adaptation.

- Webcam.

- MINI ESN Software to control the whole system.

- Cables and accessories required for a normal operation.